Project Expectations

- Develop a technology, cost, size cross matrix for 2-3 concepts that give a 6” wingspan MAV the capability to collapse/fold its wings along the body of the fuselage.

- Main emphasis should be placed on mechanical components and design, not aero-stability.  Hence, the design (except material selection) of the wing is not of significant importance, only the mechanical components for actuation of the collapsible/folding wing.

- Optimize final concept subsystems for performance, size, and weight by addressing component and material selection (i.e. plastic actuators).  It is not expected for the concept to optimize the MAV wing and fuselage materials for aerodynamics.

- Final concept selection should be able to successfully operate (stow and deploy wing structures) in static and low air speed environments (< 30 mph).

- Final cross section of MAV should fit into 3” diameter tube

- A working prototype demonstrating the collapsing wing technology is expected upon completion, including a final design demonstration at Eglin AFB, FL.

- Furnish a report detailing test results with the different materials and design outcomes as well as a budget analysis for hardware used.  Recommendations for future work should be included.

- AFRL/MNAV will be responsible for final integration of an aerodynamic concept for the subsystem into the MAV.

Preliminary Design Specifications

| Component | Specifications |
| --- | --- |
| Wing Dimensions | 6” wingspan, 3-4” width |
| Deploy/Retract time interval | 1-3 seconds |
| Fuselage dimensions | 2-2.5 in. diameter x-sec., 4-6 in. length |
| Landing System | N |
| Wing/On-board Power Source | 10-12 V |
| MAV recoverable | Y |
| Wing Material | Carbon Fiber |
| System Weight | 100-200 gm |
| Vehicle Velocity | < 25 mph |
| MAV control | Remote |
| Fuselage Material | Carbon fiber |

Project Procedures

- Background Research

  - Understanding Project Needs

    - Contact sponsor

    - Discuss project scope

  - General information search

    - Existing Micro Air Vehicles

    - Designs

    - Material

  - Consult Daniel Braley (graduate project supervisor)

- Idea Generation

  - Create concept drawings

  - Concept Matrix using Word

    - Pros

    - Cons

- Design

  - Mock fuselage using

    - Act as a base for wing deployment/retraction device

    - Dimensions relative to given wingspan

  - Wing

    - Determine storage method

      - Single, non-collapsible

      - Fold alongside fuselage, vertically

      - Bi-wing, non-collapsible

      - Fold back, top of fuselage

    - Material Selection

      - Composites

      - Weight

      - Strength

      - Stress analysis

      - Cost

  - Deployment/Retraction Device

    - Device Activation

      - Micro Actuators

        - Power consumption

        - Weight

        - Torque

        - Physical dimensions

        - Range/speed of motion

        - Cost

        - Attachment to fuselage

      - Micro Servos

        - Power consumption

        - Weight

        - Torque

        - Physical dimensions

        - Range/speed of motion

        - Cost

        - Attachment to fuselage

      - Other activation possibilities

    - Structural components

      - Linkages

        - Attachment from activation device to wing

        - Strength

        - Flexibility

        - Stress analysis

        - Cost

      - Fasteners/Fittings

        - Weight

        - Strength

        - Cost

    - Power Source

      - Determine available on-board power for activation device

        - Voltage

        - Current

      - Determine power consumption

        - Time period for wing deployment/retraction

        - Number of cycles

    - Performance

      - Ease of operation

      - Meet deployment/retraction time constraints (1-3 sec)

      - Durability

      - Reusability

Project Outline:  Attached file, Project_Schedule_Outline.mpp (Microsoft Project File)