(4) Eglin AFB – Sand Sampling Apparatus

Richard Klimas
Dan Mortensen
Ruben De Sousa
Gina Teofilak

September 30, 2008

Product Specifications:

Below are a set of initial specifications for the Eglin Sand Sampling Apparatus/method.  Product specifications define what the system must do.

- Apparatus/method will cost equal or less than $1500.

The budget for the project given by the mechanical engineering department is $1500.

- Apparatus/method must locate the projectile and the fracture sand “Shot” line within 20mm.

The precision must be within the tolerance in order to attain accurate data.

- Apparatus/method must be able to remove the projectile from the metal box target.

- Apparatus/method must remove samples of sand from the metal box target that has dimensions: 6223mm x 6223mm x 24384mm. (24.5in x 24.5in x 96in)

- Apparatus/method must be two times faster than present method of extracting sand samples.

- Apparatus/method must be durable to deal with the environment of the test area and experiment.

The environment the apparatus operates includes functioning in a warehouse setting and coping with a sandy environment

- Apparatus/method must be able to be operated with minimal steps.

- Apparatus/method must be corrosion resistant.

The environment that the apparatus is operating is harsh.

- Apparatus/method must extract a sample correctly 9 out of 10 times.

A correct sample is defined when the sample is extracted the composition has not been affected from the extraction process.

- Apparatus/method must be detachable from the metal box target.

The purpose of the specification is because if the metal box target needs to be modified or relocated the apparatus will not be damaged or be in the way.