FAMU-FSU College of Engineering 
Department of Electrical and Computer Engineering

EEL4914C/4915C – ECE Senior Design Project II

FINAL REPORT

Project title:  Autonomous Underwater Vehicle “RoboSub”

Team #: 4

Project Manager: Victoria Jefferson

Student team members:

− Victoria Jefferson, computer engineering (Email: jeffevi@eng.fsu.edu)

− Andy Jeanthenor, computer engineering (Email: jeantan@eng.fsu.edu)

− Kevin Miles, electrical engineering (Email: mileske@eng.fsu.edu) 
− Tadamitsu Byrne, mechanical engineering (Email: byrneta@eng.fsu.edu) 
− Reece Spencer, mechanical engineering (Email: spencree@eng.fsu.edu) 
− Yanira Torres, mechanical engineering (Email: torreya@eng.fsu.edu)

Senior Design Project Instructor: Dr. Bruce Harvey

Submitted in partial fulfillment of the requirements for

EEL4914C/4915C – ECE Senior Design Project II

April 12, 2011

# Executive Summary

The Autonomous Underwater Vehicle (AUV), sponsored by the FAMU-FSU College of Engineering to compete in the 14th annual AUV Competition sponsored by the Office of Naval Research (ONR) and the AUVSI organization, is a fully autonomous vehicle that will maneuver in an underwater obstacle course. Competing in the AUVSI competition course requires our vehicle to autonomously execute a sequence of tasks while remaining fully submerged in a seawater environment.

The obstacle course will be in a 16ft deep salt-water pond, located at Camp TRANSDEC in San Diego, California. Vehicles are expected to pass through an underwater gate, follow colored paths, hit two out of three buoys, avoid obstacles, either fire projectiles or drop markers at or into primary and secondary love-related figures, track an acoustic pinger and finally surface within the designate “octagon” area with a PVC structure then release it. Based on previous AUVSI competition rules, the vehicle is allocated a total of fifteen minutes to complete these varies task in the course.

The major components of the AUV have been finalized and were ordered. The motors/thrusters that were chosen were the SeaBotix SBT 150, for low power consumption. For the AUV structure, 80/20 framing was used because of its ease with adjustability. For navigation and stability control, a 9-Axis inertial measurement unit (Phidget Spatial 3/3/3) met the needs for the AUV’s underwater inertial guidance. For underwater vision, 3 LogiTech Quickcam Pro 400 web cameras are used. The top camera will be used to see the shape that the AUV is rising up in, the front-positioned camera will be used to detect any floating objects and the bottom camera will be used to see any objects on the floor of the pool. For underwater acoustic ability, four SQ-2601 hydrophones will be used and positioned accordingly for maximum sound detection. The software aspect of the project has made great progress as well. Linux is the operating system that the software engineers are using. The programming languages are C and C++ due to the ease of readability. The microprocessor for the AUV is the OMAP3530 (on the BeagleBoard), which is in the hands of the software engineers.

Currently, the team is in the final stages of the testing phase and the beginning stages of the assembly phase. The software engineers are in the process of programming the cameras, IMU, the hydrophones and microprocessor. The electrical engineer has already tested the batteries, motors and voltage regulator and is now in the assembly phase. The mechanical engineers have finalized the design for the marker dropper and camera housing and are currently in the building phase.

# For this AUV project, the main objectives were changed slightly. Originally, the main objectives were to maneuver underwater autonomously, recognize that there is a task to be done and execute that task, but due to time issues, the main objectives are to maneuver underwater autonomously and recognize the orange path. Since this is the first year performing in this competition, quality is better than quantity. As time permits, additional tasks can be added. 
Table of Contents

Executive Summary	ii

Table of Contents	ii

1 Introduction	1

1.1	Acknowledgements	1

1.2	Problem Statement	1

1.3	Operating Environment	1

1.4	Intended Use(s) and Intended User(s)	2

1.5	Assumptions and Limitations	2

1.5.1	Assumptions	2

1.5.2	Limitations	2

1.6	Expected End Product and Other Deliverables	2

2	System Design	3

2.1	Overview	3

2.2	Major Components of the System	3

2.2.1	Thrusters	3

2.2.1.1	Thruster Motor Controller	3

2.2.1.2	Thruster Power Consumption	3

2.2.1.3	Thruster Placement	4

2.2.2	Batteries Overview	4

2.2.2.1	Thruster Battery	4

2.2.2.2	Components Battery	5

2.2.3	Sensors	5

2.2.3.1	Cameras	6

2.2.3.2	Hydrophones	6

2.2.3.3	Inertial Measurement Unit	6

2.2.4	Microcontroller	6

2.2.4.1	Software	7

2.2.5	Mechanical Grabber	7

2.2.6	Marker Dropper	7

2.2.7	Frame Overview	7

2.2.8	Hull	8

2.2.9	Diagrams	8

2.2.9.1	Top View	8

2.2.9.2	Side View	9

2.3	Performance Assessment	9

2.4	Design Process	9

2.5	Overall Risk Assessment	10

2.5.1	Technical Risks	10

2.5.1.1	Leaks	10

2.5.1.1.1	Pelican Box: Vinyl Tubing Leak	10

2.5.1.1.2	Camera Housing Leak	11

2.5.1.2	Malfunctioning External Components	12

2.5.1.2.1	Camera Failure	12

2.5.1.2.2	Hydrophones Failure	13

2.5.1.2.3	Thruster Failure	14

2.5.1.3	Team Member Loss	14

2.5.1.3.1	Temporary Loss of Member	14

2.5.1.3.2	Permanent Loss of Member	16

2.5.1.4	Rules Change	16

2.5.1.5	Transportation	17

2.5.1.6	Technological Issues	18

2.5.1.6.1	Microcontroller Performance	18

2.5.1.6.2	Battery issues	18

2.5.1.6.3	Compass Interference	19

2.5.1.6.4	Drift in Inertial Measurement Unit	20

2.5.1.6.5	Camera Compatibility	21

2.5.1.6.6	Sensor or Component Communication	21

2.5.2	Schedule Risks	22

2.5.2.1	Critical Scheduling Issues	22

2.5.3	Budget Risks	23

2.5.3.1	Insufficient Equipment Funds	23

2.5.3.2	Insufficient Travel Funds	24

2.6	Risk Status	24

3	Design of Major Components	26

3.1	Batteries	26

3.2	Microcontroller	26

3.3	Adruino Board	26

3.4	Hydrophones	27

3.5	Cameras	28

3.5.1	Camera Housing	28

3.6	Inertial Measurement Unit	28

3.7	Hull	29

3.7.1	Frame	29

3.7.2	Ballast	29

3.7.3	Waterproof Connectors	29

3.8	Thruster	29

3.9	Marker Dropper	30

3.10	Mechanical Grabber	31

4	Test Plan	32

4.1	System and Integration Test Plan	32

4.2	Test Plan for Major Components	32

4.2.1	Batteries	32

4.2.2	Cameras	32

4.2.3	Hydrophones	33

4.2.4	Humidity Module	33

4.2.5	Inertial Measurement Unit	33

4.2.6	Thermocouple	33

4.2.7	Marker Dropper	33

4.2.8	Motors	34

4.2.9	Pelican Box	34

4.2.10	Waterproof Connections	34

4.2.11	Voltage Regulator	34

4.3	Summary of Test Plan Status	34

5	Schedule	36

6	Final Budget and Justification	40

6.1	Budget Justification	42

7	Conclusion	43

8	References	44

Appendix A – User’s Guide	45

A.	System Description	45

B.	Major Components	46

C.	Specifications	53

D.	Operating Instructions	54

E.	Basic Maintainance Guide	54

Appendix B – Complete Test Reports	57

Appendix C – Software	77

Appendix D – Data Sheets	82

Appendix E – Qualifications/Resumes	83

# 1 Introduction

## Acknowledgements

Team 4 would like to thank Northrop Grumman and Harris Corporation, as well as the FAMU/FSU College of Engineering Electrical/Computer and Mechanical Engineering departments for monetary contributions towards the project. The team would also like to thank ARM Holdings for donating a processor and microcontrollers for the automation of the vehicle, and finally we would like to thank Dr. Bruce Harvey of the FAMU/FSU College of Engineering for his guidance and advice. Without these contributions, the design and construction of the AUV would not be possible.

## Problem Statement

The Association for Unmanned Vehicle Systems International (AUVSI) Foundation and the U.S. Office of Naval Research (ONR) has designed a competition to advance the development of Autonomous Underwater Vehicles (AUVs) by challenging a new generation of engineers to perform realistic missions in an underwater environment. The competition requires the design of a vehicle that operates autonomously in a self-contained unit. The AUV will be deployed in a large salt-water pool at Camp Transdec, CA, which has a maximum depth of 16 feet. The rules and competition objectives are similar to the 2010 AUVSI competition and can be found online at www.auvsifoundation.org/AUVSI/FOUNDATION. The competition has several objectives to complete, which require an optical sensor, for color and shape recognition, and an acoustic sensor for the identification and location of a pinger.

The AUV is battery powered and operates using an ARM processor/controller and an Audrino Board, which interprets all data collected by the sensors to control the four Seabotix SBT-150 motors and directional heading of the vehicle. It is designed to be as small as possible to save on weight and spatial dimension so that any last minute changes can be made without worry of disqualification due to physical constraints. Three Logitech QuickCam Pro 4000 cameras are used for color and shape recognition, which allow the AUV to complete most of the objectives found in the competition rules. Four Sensor-Tec SQ26-01Hydrophones are utilized to identify and locate an acoustic pinger at the bottom of the pool, which marks the rescue object. A mechanical grabber attached to the AUV provides a means to bring the rescue object to the surface while constraining the object in 3 degrees of freedom. The four Seabotix SBT-150 motors will provide propulsion, two motors facing the rear of the AUV will provide lateral and turning movement, and two motors located on the front and back of the AUV will provide vertical translational control. The programming language has been narrowed down to C and C++; both languages are used and the operating system is Linux.

## Operating Environment

The AUV operating environment is a salt-water pool at Camp Transdec, California. The maximum depth of the pool is 16 feet and it will be hoisted into the pool and slowly lowered to avoid damage. The AUV has 15 minutes to complete all tasks mentioned above which traverses a distance of 50 feet. The salinity of the water is accounted for in the design process in order to establish 0.5% buoyancy and to ensure that all components are corrosion resistant.

## Intended Use(s) and Intended User(s)

The AUV is intended to compete in the AUVSI 14th annual competition, which establishes very specific guidelines and is therefore designed and programmed to operate in a salt-water pool. The vehicle is an example of how robotic submersibles complete tasks in harsh environments and can also be adapted for use in a zero gravity environment. The AUV is to be used only by team 4 and operation requires a push of a button and a pool since the vehicle is autonomous.

## Assumptions and Limitations

### Assumptions

- The AUV is be completely autonomous

- There is a clearly identifiable kill switch to shut down the AUV

- The AUV operates in a salt water pool

- The AUV is battery powered

- The AUV detects color, shape, and sound

- The AUV automatically shut down after 15 minutes for competition

- The AUV has hoist points so that it can be slug and lowered into the water

- The AUV has 4 thrusters to provide movement

- The AUV is able to read and write from shared memory

- The AUV has an inertial guidance system to detect changes in orientation or position

### Limitations

- The AUV has to be less than 6ft x 3ft x 3ft in size

- The AUV has to be less than 110 pounds

- The AUV has 15 minutes to complete all tasks

- The project budget is $9,500

- The AUV must utilize a ARM processor/controller

- The markers on the AUV should not exceed 6.0in x 0.5in x 0.5 in

- The battery should not have an open source voltage exceeding 60VDC

## Expected End Product and Other Deliverables

The end product is a vehicle that is fully autonomous and submersible, which can complete all objectives outlined in the AUVSI competition rules and mission. Team 4 provides an AUV that is compact, lightweight, and competitive. The product requires no input from the user to function properly, but it must be placed in an aquatic environment. A team journal paper is required with the submission of the AUV, which will highlight the mechanical, electrical, and computer systems, all hardware and software utilized, and the testing procedures of the final product. The final product will be available at the end of the Spring 2011 semester with minor adjustments and debugging continuing up to the competition day, which is July 12th, 2011.

# System Design

## Overview

The design of the Autonomous Underwater Vehicle is similar to a rectangular submarine with several additions in order to complete the tasks required for the competition. The frame was constructed with 80/20 aluminum and the hull is a pelican box in the middle of the frame. The design is 7.5” X 24” X 28”. All of the electronics are housed in the watertight hull. Refer to Appendix A for the user’s manual that will and/or assist with any clarification needed.

## Major Components of the System

### Thrusters

There are a total of four Seabotix SBT 150 thrusters. These motors are engineered with additional components, which have made them an excellent choice for the AUVSI team. The motors have an attached propulsion fan blade, as well as a motor controller, which fits into its cylindrical hull. Each motor weighs approximately 1.5 lbs (686 grams) when dry and -0.866 lbs in fresh water. Each motor is connected with an in-line 6A max barrel fuse, as well as a 10A rated diode (diode chosen for availability) to prevent back EMFs from the motor.

#### Thruster Motor Controller

The SBT 150 comes with motor controllers already connected with the internal motor. The controllers call for a nominal voltage of 28V DC. However, the motors run with as low as a 20V input into the controllers. The motor controllers have built in voltage regulators, which give an actual supply voltage of 19.1V DC to the internal motor. When the motor controller receives less than 20V DC, or greater than 30.1V DC, it shuts itself off automatically to preserve the integrity of the motor. The wiring configuration for the motor controller calls for Vcc, GND, Data, and Clock. The VCC and GND utilize a 14-gauge AWG wire, while the data and clock inputs have 18-gauge AWG wire.

#### Thruster Power Consumption

The max amperage consumption is 5.8A (30 second duration) as well as max continuous amperage consumption of 4.25A. The maximum power consumed by each motor is 174W. The motor is capable of handling higher currents of up to 6A, but will drastically shorten the life of the thruster when ran continuously in this mode. High current burst of less than 1 minute may be used, provided that the running average current is kept at 4A MAX. This prevents the windings of the motor from building excessive heat. All of these numbers are given with a 10% tolerance on the motor specifications sheet and all data listed above is per motor.

#### Thruster Placement

Since the hull is filled with water, the AUVSI team has four motors with attached thrusters to control the direction and depth of the submarine. A thruster is located on the left and right side of the submarine to control the turning as well as forward and reverse propulsion. Additionally, there are two thrusters on the bottom of the unit, one on the front as well as one on the back. These are placed as such to alienate problems in the weight distribution of the vehicle.

### Batteries Overview

Many batteries were considered for the design, but due to its light weight it was decided that the lithium-ion is the ideal candidate for the job. There is one power unit consisting of two batteries in series on board the AUV to power its components. One main power supply is used for the thrusters, while a switching voltage regulator steps down the power to 5V for all other electrical components on board. These batteries are rechargeable and have a capacity to run the system for at least a complete hour.

#### Thruster Battery

The batteries chosen for the thrusters are custom Lithium Ion batteries. Two 14.4V DC batteries are placed in series for a total of 28.8 V DC. The batteries come with a built in protection circuit module which maintains a voltage of between 20.8V and 30.6V. Anything outside of this range, the battery disconnects from the circuit. The PCM also protects from over drain if the amperes exceed anything greater than 40A, however that will be unlikely. The charger for each battery takes 10.1 hours to supply a full recharge. After a full charge, it takes an additional 30 minutes for the PCB to evenly distribute the cells inside the battery.

#### Components Battery

The rest of the components on board are powered with the same two 14.4V Lithium Ion batteries, connected to a Hercules Switching Voltage Regulator (SVR). The SVR takes an input from 15-45V DC, and outputs 5.3 V DC. This output provides current up to 6 amps for a tested 6 hour, 150 degrees Fahrenheit environment. This supplies USB 2.0 specification power to the microprocessor, which runs off of 5.3V and consumes a maximum of 2A. The system is simplified and all additional components such as the cameras are powered by the USB connection to the microprocessor. All components are connected with an inline fuse—rated at its peak amperage consumption, and certainly no more than 2A.

### Sensors

Due to the preliminary requirements of this year’s competition, there is a need of several sensors for the AUV. Some are specifically needed for specific tasks, while others will just be for general maneuverability and not are necessarily associated with a specific task.

MARKER DROPPER

MOTORS

MOTOR CONTROLLERS

ARDUINO BOARD

CAMERA #3

CAMERA #2

CAMERA #1

IMU

USB HUB

BEAGLEBOARD (ON-BOARD COMPUTER)

Computer System Structure

HYDROPHONES

HYDROPHONE BOARD

#### Cameras

The cameras on board the AUV have two main functions in order to complete the tasks for the competition. The first is color recognition. An orange colored 6” wide sheet of PVC serves as the path where the cameras recognize the orange color among the other colors. Additionally, the cameras are housed in a watertight casing, due to the nature of the operating conditions of the vehicle. Secondly, the camera recognizes various shapes within the obstacle course. For example, it recognizes a silhouette of X’s and O’s among other objects. We also use the shape recognition to locate the path as well, but that is yet to be determined. The camera’s data is input into the microcontroller for data analysis.

#### Hydrophones

A part of the competition includes a “pinger” secured to the bottom of the pool. Since the pinger will broadcast signals throughout the pool, the objective is to recognize the signal, then determine its location. The design consists of four hydrophones that assist in determining the location of the pinger. The hydrophone’s data is input into the microcontroller for data analysis which then adjusts the position of the AUV accordingly.

#### Inertial Measurement Unit

For the competition, it is obvious that the AUV requires stability and control before the AUV even begins completing any given task. The inertial measurement unit (IMU) assists the team in stability control. The IMU’s data is input into the microcontroller for data analysis.

### Microcontroller

The microcontroller serves as the entire functioning “brain” of the AUV. The microprocessor takes input from the different sensors, analyzes the data, and then sends output signals to the motors, mechanical grabber, and marker dropper as needed. The motors are powered through the batteries; however the direction and speed are controlled by pulse width modulation. The microcontroller has an ARM processor and was generously donated for the project. The board runs off of 5 volts with a max current draw of 1 amp.

#### Software

To program the microcontroller, C++ and C programming languages will be used. The operating systems will be Linux. The software engineers have updated the pseudo code for performing the tasks and how the sensors will send information to the microprocessor based on the preliminary rules that were released late November. The software engineers are in the process of working with OpenCV and the IMU. For the sample of the pseudo code for completing specified tasks, see Appendix C.

### Mechanical Grabber

The design of the grasp and release mechanism is located at the bottom of the AUV. The design of the mechanical grabber consists of a mechanical claw attached to a solenoid that attaches to a specified object in the water and brings it to the surface within the octagon.

### Marker Dropper

The marker dropper is an aluminum housing which contains a servo-motor, which turns in both a clockwise and counter-clockwise direction and two small steel spheres, which act as the markers. An aluminum arm holds the markers in a recessed area of the vehicle. When the servo-motor is activated the aluminum arm rotates in one direction and one of the markers drop to the bottom of the pool, when the arm rotates in the opposite direction the unit drops the second marker.

### Frame Overview

The frame is a rectangular design with weight and simplicity in mind. The outer part of the frame is constructed of 80/20 Aluminum. In the next section, you will find an image of the AUV. The reason for the 80/20 Aluminum is for easy adjustability of any component connected to the frame.

### Hull

The hull consists of a watertight Pelican box, which is placed on moveable rails inside the frame. The hull has moveable rails to exercise weight stability. The pelican box is 16”x12”x7.5” (LxWxH). The pelican box houses the microcontroller, batteries, humidity sensor, temperature sensor, and all of the on board power management components. All other electrical components (cameras, marker dropper, and mechanical grabber) located outside of the Pelican box are connected through a vinyl tube that is clamped onto a brass fitting that is attached to the side of the Pelican box, allowing all wiring to pass directly through the Pelican box and into the microcontroller.

### Diagrams

#### Top View

#### Side View

## Performance Assessment

The objective of this project is to design, construct, and program a functional Autonomous Underwater Vehicle (AUV) capable of winning the AUVSI RoboSub Competition.  The AUV operates autonomously, maneuvers underwater, and has abilities to read colors and recognize pining signals. Team 4’s AUV meets the needs and requirements for the project, specified by the official competition rules.

Assessing how the system would meet the needs and requirements for the project took hard work by the members of Team 4. The AUV designed by Team 4 met the needs and requirements due to the selection of equipment and software. The AUV maneuvers underwater with its propulsion systems comprised of four (4) motors/thrusters. Underwater vision is accomplished by the use of three (3) cameras, positioned frontally, upwardly and downwardly to maximize the view of the scenery. In order to recognize the underwater pinger, the use of four (4) hydrophones –2 positioned vertically and 2 positioned horizontally, achieve this. An inertial measurement unit is used for navigation and stability control. In order to power the AUV, the right battery was critical. Team 4 has received the batteries, and has been tested to make sure the batteries are indeed the right battery for the job. To meet the weight requirement, the team’s choices of equipment took into account its weight and to meet the size dimension limitation, the size dimensions of the equipment were also taken into account.

## Design Process

At the beginning of this project, there were a lot of unknowns since this project was starting from scratch. In a mere six months, the majority of those unknowns are now known. After a team was put together, numerous decisions still remained. Decisions that needed to be made included how was the AUV going to look, how were we going to raise more money than what we had already, and what would each member’s task be within the project. After these decisions were made, more complex decisions arose that required more thought such as what equipment will the team use, will it be in the budget, if not, what alternatives will be feasible and will it affect the intended goals. These decisions required a lot of research, but the team as whole has successfully decided on all of the AUV’s major components.

Team 4 has made great progress including finalizing and purchasing equipment. Major components such as motors/thrusters, the hydrophones, framing, batteries have been chosen, purchased, and received. For the motors/thrusters, four SeaBotix SBT150 motors are used, which have been purchased and are currently going through the testing phase. Two (2) SensorTec SQ26-01 hydrophones were ordered for testing and the other two hydrophones are in the process of being ordered. Team 4 chose 80/20 framing, which has already been received, for the structure of the AUV. The frame for the AUV has since been constructed and is going through testing. The team still has to make final decisions on the interfacing for the microprocessor and sensors. Unfortunately, due to some compatibility issues—some of the original equipment chosen such as the cameras, will not suit the needs for this competition.

Even though the majority of the equipment is finalize and purchased, the journey is only midway—the remainder of the time will be vital to the AUV’s success. As the last remaining equipment begins to arrive, the team needs to finish testing them and complete the building phase. The microprocessor was already supplied to the team, but the software engineers have a tough decision ahead of them. The software engineers are in the process of writing code and seeing how everything will communicate with each other as well as writing the code for the motors and servo motors. The electrical engineer in the team is in the process of assembling the major components to make sure the batteries can last when being used to power everything together.

## Overall Risk Assessment

### Technical Risks

#### Leaks

##### Pelican Box: Vinyl Tubing Leak

| Risk | Improper integration of vinyl tubing |
| --- | --- |
| Probability | Low |
| Consequence | Catastrophic |
| Strategy | Extensive testing before integration of electrical components |
| Still relevant? | Yes |
| Occurred? | No |
| If yes, how was it resolved? | N/A |

If the vinyl tubing is integrated inappropriately through the pelican box, the consequence would be catastrophic, as this would result in the exposure of corrosive salt water to water sensitive electronics and devices. The probability of this occurring is low. To prevent detrimental damage to several key components of our vehicle the group, after the vinyl tubing have been integrated into the pelican box and before all of the electrical components have been assembled the pelican box will be extensively tested to ensure there are no leaks where the tubing has been implemented. The empty pelican box will be submerged to the operational depth several times to simulate the operational requirements of the competition. The box will be examined to ensure that no leaks have occurred during testing, once confidence in the integration of the vinyl tubing has been established the electrical components will be assembled inside. This risk is still relevant because besides passing the testing phase, the AUV will still have to maintain its waterproof state throughout the competition.

##### Camera Housing Leak

| Risk | Leak of camera housing |
| --- | --- |
| Probability | Moderate |
| Consequence | Serious |
| Strategy | 1. Extensive testing and analysis before integration of camera |
| Still relevant? | Yes |
| Occurred? | No |
| If yes, how was it resolved? | N/A |

There is a moderate probability of the camera housing fracturing or failing, inducing a leak, which would result in the malfunction or permanent damage of one or more cameras.  The occurrence of this risk would be serious because the three cameras are used in several of the competition tasks. To avoid the occurrence of this risk the AUV team will perform extensive analysis and testing on the camera housing to ensure it is effective at eliminating the threat of the camera's exposure to moisture. If the camera housing does fail and damages a camera the AUV team has minimization strategies and contingency plans to resolve these issues. This risk is still relevant because besides passing the testing phase, the camera housing will still need to maintain its waterproof state until the completion of the competition.

#### Malfunctioning External Components

##### Camera Failure

| Risk | Failure, damage or malfunction |
| --- | --- |
| Probability | Moderate |
| Consequence | Serious |
| Strategy | 1. Develop alternative camera configuration for lack of camera 2. Replace damaged cameras. |
| Still relevant? | Yes |
| Occurred? | No |
| If yes, how was it resolved? | N/A |

The probability that a camera will fail, be damaged, or malfunction is moderate. If a camera is damaged before the competition, the team will examine the warranty of the camera to determine if the part can be returned and a replacement may be obtained in a timely manner.   If the warranty is void, the immediate purchase of the replacement part is necessary. If the issue with the camera is repairable, the team will attempt to repair it.

If time does not permit the purchase of cameras, the remaining cameras may be reconfigured to accomplish some of the requisite tasks.  If only one camera is unusable and time or budget constraints do not permit the purchase of replacement the other two cameras may be reconfigured with one camera on the front and bottom of the vehicle may be used. The camera on the top is needed to find an octagon and ascend within it; this task may be completed simply with a camera on the front of the vehicle by tilting the device at an angle to search for the octagon. The accuracy and precision of the ascension of the vehicle inside the octagon may be compromised in this case but will accomplish the requisite task.  If two cameras are unusable then the remaining camera may be placed on the front of the device and will need to find the octagon and also be able to find the rescue object on the floor of the pool, using the same technique of tilting to view the area above and below the vehicle. During competition if one or two of the cameras malfunction, the remaining camera will be utilized to perform the remainder of the tasks. If budget does not permit purchase of additional cameras, see Budget Risk in Section 2.5.3.

If cameras are damaged during competition the vehicle will utilize the remaining cameras to perform the remainder of the tasks to the best of its capabilities. If the top and/or bottom camera are damaged the remainder of the tasks can be completed purely with the front camera. This can be accomplished by tilting the vehicle up and down to search for targets and the octagon. If the front camera is damaged the vehicle may still find the rescue object and rise inside of the octagon. This risk is still relevant because besides passing the testing phase, the cameras will still need to maintain its operational functions throughout the competition.

##### Hydrophones Failure

| Risk | Failure of one or more hydrophones |
| --- | --- |
| Probability | Low |
| Consequence | Serious |
| Strategy | 1. Analyze damage, return or repair if possible 2. Reconfigure to accommodate for lack of single hydrophone. |
| Still relevant? | Yes |
| Occurred? | No |
| If yes, how was it resolved? | N/A |

The probability that a hydrophone will fail is low because they are developed for much more intense conditions than those observed during out competition. If one of the hydrophones fails the consequence will be tolerable but if more than one hydrophone fails the consequence will be serious. To prevent failure the team will be careful while integrating the hydrophones to vehicle, to ensure damage does not occur to any part of the component. Care will also be examined when testing and competing to ensure the parts are not damaged. If the hydrophones do fail the team will examine the warranty to determine if the part can be returned and a replacement may be obtained in a timely manner. If the warranty is void, the immediate purchase of the replacement part is necessary. If the issue with the hydrophone is repairable, the team will attempt to repair it. If time does not permit the purchase of replacement hydrophones, the remaining hydrophones may be reconfigured to accomplish the requisite task to the best of the vehicles ability. If only one hydrophone is damaged the remaining three hydrophones will be reconfigured to appropriately triangulate the position of the pinger. If two hydrophones are damaged the remaining two will be reconfigured to identify the depth and distance of the pinger by aligning the hydrophones vertically. The distance/direction of the pinger will be established by the change of the magnitude of the voltage output from the hydrophones, if there is a decrease in the magnitude of the voltage output the vehicle will be able to interpret this as it is going in the wrong direction while if there is an increase in the voltage it will know it is going in the right direction. The vertically placed pingers will be utilized to identify a time delay, which will indicate the depth of the pinger. Once the device is spotted the cameras will be able to complete the task. If budget does not permit purchase of replacement hydrophones see Budget Risk in section 2.5.3. This risk is still relevant because besides passing the unit testing phase, the hydrophones will still need to maintain its operational functions throughout integration testing and until the competition.

##### Thruster Failure

| Risk | Failure of one or more thrusters |
| --- | --- |
| Probability | Low |
| Consequence | Serious |
| Strategy | 1. Analyze damage, return or repair if possible 2. Reconfigure to accommodate for lack of single thruster. 3. Float to surface for safe recovery. |
| Still relevant? | Yes |
| Occurred? | No |
| If yes, how was it resolved? | N/A |

There is a low probability of the failure of the thrusters; failure of thrusters would have a serious consequence because they are utilized for the vehicle maneuverability, which is required to complete all of the tasks.  If a thruster is damaged before competition, the team will examine the warranty to determine if the part can be returned and a replacement may be obtained in a timely manner. If warranty is voided, the immediate purchase of the replacement part is necessary. If the issue with the thruster is repairable, the team will attempt to repair it. If time does not permit the purchase of replacement parts, the remaining thrusters may be reconfigured to accomplish some of the requisite tasks. If only one thruster is damaged the remaining three may be reconfigured without losing functionality, utilizing two thrusters to turn the vehicle and one to descend and maintain submersion. If thrusters fail during operation there will be an emergency system that will increase the buoyancy of the device and cause it to rise to the surface for safe recovery. If budget does not permit purchase of additional thruster see Budget Risks in section 2.5.3. This risk is still relevant because the completion of competition demands on the AUV’s maneuverability. If the thrusters fail before or during the competition, the AUV will no longer be able to compete.

#### Team Member Loss

##### Temporary Loss of Member

| Risk | Temporary loss of team member |
| --- | --- |
| Probability | Moderate |
| Consequence | Serious |
| Strategy | 1. Temporarily reallocate team member's work so that it is accomplished by deadlines. 2. Seek assistance from interested individuals in the engineering school to take on parts of team member's work and work with them when they return. 3. Keep member up to date on advancements of the project. |
| Still relevant? | Yes |
| Occurred? | Not Yet |
| If yes, how was it resolved? | N/A |

There is a moderate risk that a team member will be temporarily lost as a result of sickness, injury or absent due to unforeseeable circumstances and the consequence would be serious because the individual team member will be the most experienced with their technical focus area in the project. In the event of this occurrence adjustments will be made to compensate for the temporary loss of the member. The team member’s documents will be gathered and their work will be split among the team based on individuals’ areas of the strength. To avoid this issue, the AUV team is currently in the process of creating an AUVSI registered student organization to recruit interested students to assist in our project. These students will be kept up to date with the project so that if a situation arises they will be sufficiently knowledgeable about the topic and can effectively complete it in an acceptable amount of time. If there are sufficient amount of involvement each member will develop a committee compiled of students interested in their focus, this committee will be extremely knowledgeable in the inner workings of the committee leaders part of the project so that if an issue arises we will have interested and committed students ready to accommodate for the temporary loss of a team member.  While the member is unavailable to work they will be kept up to date on advancements of the project so that when they return they can pick up where they left off and continue work effectively without being off schedule. This risk is still relevant because since the competition is in the summer, some of the team members will be graduating or will have internships, which can or will prevent them from participating in the competition and/or the completion of the AUV.

##### Permanent Loss of Member

| Risk | Permanent loss of team member |
| --- | --- |
| Probability | Low |
| Consequence | Serious |
| Strategy | 1. Seek assistance from interested individuals in the engineering school to continue team member's work. 2. Reallocate team member's work to other team members. |
| Still relevant? | No |
| Occurred? | No |
| If yes, how was it resolved? | N/A |

There is a low risk that a team member will be permanently lost as a result of unforeseeable circumstances and the consequence would be serious because the individual team member will be the most experienced with their technical focus area in the project. In the event that a team member will not be able to return to complete the project adjustments will be made to compensate for the permanent loss of that member. The team member’s documents will be gathered and their work will be split among the team based on individuals’ areas of the strength. To avoid this being a major issue, the AUV team is currently in the process of creating an AUVSI registered student organization to recruit interested students to assist in our project. These students will be kept up to date with the project so that if a situation arises they will be sufficiently knowledgeable about the topic and can effectively complete the desired tasks. An ambitious and committed student will be reallocated all of the tasks associated with lost team member and, if sufficient participation from students, will lead a committee to assist in completion of said tasks. This risk is no longer relevant because the semester is almost over.

#### Rules Change

| Risk | Drastic rules change in late February |
| --- | --- |
| Probability | Low |
| Consequence | Catastrophic |
| Strategy | 1. Only prepare for objectives that have been consistently repeated 2. Reanalyze the budget, schedule, and system to determine feasibility of including more of the objectives required for competition. |
| Still relevant? | No |
| Occurred? | No |
| If yes, how was it resolved? | N/A |

Since the preliminary rules for this year’s competition have been released there is a low probability that there will be substantial changes to the rules and objectives. The result of a drastic rules change would be catastrophic if we based our design on the previous year. To reduce the consequence of this risk the AUV team has researched several competition rules from previous years and included components and objectives that have consistently repeated to ensure that we do not include things that will not be required. If there is a drastic change in the rules the team will have to abandon any requirement that is not required to qualify for the competition and begin to reanalyze the budget and schedule to determine which components from the new rules can be included in the system in a timely manner, refer to sections 2.5.2 and 2.5.3 for budget and schedule risks. This risk is no longer relevant because the preliminary rules were released in early December, which eliminated this risk.

#### Transportation

| Risk | Vehicle damage due to transportation |
| --- | --- |
| Probability | High |
| Consequence | Catastrophic |
| Strategy | 1. Carefully package the vehicle. 2. Drive to California with the vehicle and ensure it is properly secured throughout the trip. |
| Still relevant? | Yes |
| Occurred? | N/A Competition has not arrived yet |
| If yes, how was it resolved? | N/A |

The probability of the vehicle being damaged during transportation is high due to the large distance that needs to be travelled the consequence would be catastrophic because it would result in the inability to compete. To reduce this risk the team will be driving to California with the vehicle instead of shipping it or taking it through an airline. The vehicle will be meticulously packaged and secured throughout the trip to further ensure the devices safe arrival in San Diego, California. This risk is still relevant because the competition has yet to come. Transporting the AUV from Tallahassee, FL. to San Diego, CA is a huge risk for the team, which will be analyzed closer to the time of competition.

#### Technological Issues

##### Microcontroller Performance

| Risk | Microcontroller not performing task properly |
| --- | --- |
| Probability | High |
| Consequence | Catastrophic |
| Strategy | Test and debug |
| Still relevant? | Yes |
| Occurred? | No |
| If yes, how was it resolved? | N/A |

There is a high probability that the microcontroller will not perform tasks properly, during competition the consequence would be catastrophic. To prevent this event during competition the entire AUV system will be tested and debugged repeatedly to ensure that it is capable of performing all desired tasks effectively. If the microcontroller is not performing a desired task, that specific area of the software will be reviewed. If no solution is found the software for the desired function will be rewritten and reintegrated into the main program. If drastic measures are required the entire program will be rewritten. This risk is still relevant because the programming for this project is crucial for the success of the AUV in the competition. Even though this risk has not occurred, the software engineers constantly are watching the performance.

##### Battery issues

| Risk | Battery over-discharging, overcharging, shorting terminal |
| --- | --- |
| Probability | Low |
| Consequence | Catastrophic |
| Strategy | Purchase battery with built in protection module |
| Still relevant? | Yes |
| Occurred? | No |
| If yes, how was it resolved? | N/A |

The risks associated with the battery over-discharging, overcharging, shorting terminal is low but the consequence is catastrophic as it would result in deterioration of the electrodes, reduced battery life, and/or a fire. To prevent the occurrence of any of these issues the AUV team purchased a battery with a built in protection circuit module. This protection circuit module keeps the 14.8V Li-Ion battery pack from overcharging and over-discharging while balancing each cell at a maximum of 4.22V/cell. This risk is still relevant because besides passing the testing phase, the AUV is battery powered so if the battery encounters any problems—the AUV will not run.

##### Compass Interference

| Risk | Magnetic interference of compass |
| --- | --- |
| Probability | High |
| Consequence | Serious |
| Strategy | 1. Perform testing to ensure magnetic interference from electronics is minimal. 2. Utilize inertial measurement unit to compare values output by compass to ensure interference at competition does not mitigate performance and effectiveness |
| Still relevant? | Yes |
| Occurred? | N/A |
| If yes, how was it resolved? | N/A |

There is a high probability that the compass will experience magnetic interference, either from electrical components in the pelican box or from materials in the pool. To ensure that the magnetic interference from the electronics is not substantial and does not alter the compasses capabilities the compass will be tested with the electronics. Since the composition of the pool is unknown there may be magnetic material that could also interfere with this device, to ensure that the reading will in fact be accurate the gyroscope used to measure yaw will be related to the compass. When the device is placed in the water the two measurements will be in sync: the gyroscope will be calibrated to an initial angle, which will correlate to a certain compass heading. This relationship should remain consistent throughout the competition. If there is an error with the compass as a result of some interference these to values will no longer be in sync. Thus the gyroscope will act as a check for the compass, if the vehicle determines that these two values are not in sync then the output from the compass will be ignored and it will complete the remainder of the tasks without the compass. This risk is still relevant because during the testing phase—the compass was not by anything that would cause a magnetic interference. Once the integration test of the AUV is done, this risk will be reevaluated.  Also, the team is not sure if the Camp TRANSDEC pool contains any material that would cause interference.

##### Drift in Inertial Measurement Unit

| Risk | “Drift” in Inertial Measurement Unit |
| --- | --- |
| Probability | Low |
| Consequence | Serious |
| Strategy | 1. Recalibrate IMU before use. 2. Test to ensure substantial error accumulation does not occur during competition time. |
| Still relevant? | Yes |
| Occurred? | N/A |
| If yes, how was it resolved? | N/A |

After prolonged use inertial measurement units (IMU) are subject to Abbe Error, accumulated error, which leads to drift in measurements. To reduce the accumulation of error the IMU will be recalibrated before competition, practices and testing. To prevent serious consequences during competition the system will be tested several times and the measurements output from the IMU will be analyzed to determine if a substantial amount of Abbe Error is induced during the length of time during competition, fifteen minutes. If substantial accumulation does occur the team will need to determine at what point it begins to occur and the amount of error. This information will be utilized to determine a correction factor to be implemented to the values output by the IMU after this point to ensure that the drift is accounted for and does not cause major errors and instabilities. This risk is still relevant because recalibration was not been done yet.

##### Camera Compatibility

| Risk | Compatibility with programming language |
| --- | --- |
| Probability | Moderate |
| Consequence | Serious |
| Strategy | 1. Research camera compatibility, if possible 2. Test immediately once received. 3. Return camera and purchase known compatible camera |
| Still relevant? | Yes |
| Occurred? | Yes |
| If yes, how was it resolved? | Changed from Laptop to Desktop |

One type of camera will be utilized in the AUV design. Previous research was conducted on the previous camera that was chosen: Unibrain Fire-I, but due to compatibility issues—there is a possibility that this camera will not be used in the design. From this research it was determined that the Unibrain Fire-I is Linux compatible. Further research showed that the cameras needed to be UVC compatible as well, which the Unibrain Fire-I did not specify.  Before that camera was ordered, the team replaced the Unibrain Fire-I camera with a LogiTech QuickCam Pro 4000. Once that camera is received, it was tested immediately to establish compatibility. Even though it said it was compatible with Windows Vista, the software engineers encountered compatibility issues. Due to timing, the camera was not returned. Software engineers changed venues—from a laptop to desktop where the operating system was Windows XP. This method caused a delay with the cameras, but it was the cheapest and quickest way to resolve this situation.

##### Sensor or Component Communication

| Risk | Errors in sensor-microcontroller communication |
| --- | --- |
| Probability | Moderate |
| Consequence | Serious |
| Strategy | 1. Reevaluate connections 2. Reevaluate programming 3. If possible, remove function 4. Explore alternative sensors as replacements |
| Still relevant? | Yes |
| Occurred? | N/A Interfacing phase has not started |
| If yes, how was it resolved? | N/A |

If the sensors are not communicating with the microcontroller correctly, the wired connections and programming will be reevaluated. If no solution is found the team will seek faculty assistance to establish the issue and determine a resolution. If that does not work the specific function will be excluded until further notice, if the sensor is necessary for successful completion of tasks an alternative sensors will be explored and purchased if the budget allows. If not, see Budget Risk in section 2.5.3. This risk is still relevant because the integration of all of the sensors has not been completed yet.

### Schedule Risks

#### Critical Scheduling Issues

| Risk | The team is critically behind schedule |
| --- | --- |
| Probability | High |
| Consequence | Severe-Catastrophic(depending on severity of scheduling issues) |
| Strategy | - Reallocate man power - Work Overtime - Abandon Function/Task |
| Still relevant? | Yes |
| Occurred? | Yes |
| If yes, how was it resolved? | Team is working overtime; some objectives scrapped |

In the event that production is vastly behind schedule the project manager is tasked with evaluating team performance. The probability of this occurring is very high and the consequences are not severe early on. As time progresses the problem will become worse and drastic measures will have to be taken. The team can get behind for a number of reasons. Some can be avoided and others may be inevitable, such as the fact that the official rules for this year’s competition have yet to be released. The project manager will ensure that new methods of production will be put in place to increase efficiency. Every team member is required to work overtime without pay as well as extra hours on weekends. Depending on the severity of the issue sections of the design that are not very important will be scrapped. In an effort to present the sponsors and the FAMU/FSU College of Engineering with a finished product, portions of the design that are not feasible at that point will be delayed until further notice. The team will have to decide what portions of the design to continue to pursue and what to abandon. The team will be required to provide extensive documentation as to why this problem occurred and what can be done to prevent this problem for the next team working on the AUV. Compared to where the team is supposed to be, the team has encountered critical scheduling issues. Team is working overtime to complete the testing phase and transition into the building/assembly phase.

### Budget Risks

#### Insufficient Equipment Funds

| Risk | More things needed to be purchased but no funds are available |
| --- | --- |
| Probability | Moderate |
| Consequence | Severe |
| Strategy | - Extensive research for pricing and efficiency of parts as well as careful attention to the welfare of all components - Fundraising - Sponsorships |
| Still relevant? | Yes |
| Occurred? | No |
| If yes, how was it resolved? | N/A |

The completion of this project hinges upon the ability of the team to make engineering related decisions. The team is working with a fixed budget, which means that there are some monetary risks associated with bad choices. The choice of parts will be decided by both the cost and the efficiency of the equipment. The current budget comes with a moderate risk of a deficit in funds. This risk can occur when if there are insufficient funds and the following events occur: parts malfunction, wrong parts have been chosen and if changes are made to the competition rules requiring new parts to be purchased. These problems can be prevented by properly estimating the budget from the beginning and making sure all the parts are cost efficient. The problems can be resolved by attempting to receive more sponsorship from corporations in which they will receive an incentive for their donation. Some sort of fundraising may also be considered to provide income in case additional funds are needed. The last option will be to either pool money together and open a bank account for the project or open a credit card account for the project. This risk is still relevant because if any equipment is damaged, the team may not have enough money to replace them especially the larger valued pieces such as the motors and batteries.

#### Insufficient Travel Funds

| Risk | Funds are not available for the team to travel to the competition |
| --- | --- |
| Probability | Very High |
| Consequence | Catastrophic |
| Strategy | - Fundraising - Sponsorships |
| Still relevant? | Yes |
| Occurred? | N/A Competition has not arrived yet |
| If yes, how was it resolved? | N/A |

The purpose of this project is to enter the AUVSI Robosub Competition, which is located in San Diego, California. Upon completion of our design we plan to enter this year’s competition. This risk is dependent upon whether there are funds available for the team to travel to California. The current travel budget is estimated to account for the travel expenses of the entire team. This risk is very high because the team does not have any funds for the current travel budget. The consequence of this risk is catastrophic because the purpose of this project is to design a robot that will represent the FAMU/FSU College of Engineering at the AUVSI Robosub Competition. Obviously if there are no travel funds the team will not compete. Doing fundraisers and attempting to obtain sponsorships can avoid this. Also, reservations should be made as soon as possible to reduce the costs and the entire team may not travel to reduce the costs some more. The worst-case scenario would be that no funds are available and steps will be taken to ensure that the next FAMU/FSU COE AUV team will be able to travel to the next competition. This risk is still relevant because a realistic transportation budget has not been finalized. Once the transportation budget is completed—this risk will be evaluated again to see if this risk will prevent the team from competing.

## Risk Status

In this project there were technical, budget, and schedule risks that the team has encountered or that are still relevant. Most of these risks have a moderate to low probability of occurring but in the case that issues do occur the risks are well understood and the team has developed a strategy to handle these dilemmas as they are encountered. There are some risks that are of higher concern, including the potential for damage of our vehicle during transport, and compass interference because of their higher probability of occurrence. To ensure that these issues do not induce serious or catastrophic consequences the team has taken the necessary precautions to lessen the probability of occurrence and/or lessen the severity of the associated consequences. There are also certain risks in which the team needed additional help to manage, for example risks associated with sensor and microcontroller communication. Because of the vast amount of variables and issues associated with certain aspects and components of the vehicle, additional help from faculty to identify and solve the problem in a timely and effective manner was needed and appreciated.

# Design of Major Components

In order for the AUV to function properly it must process data, maneuver underwater, and perform mission tasks, which require a power source that is capable of powering all the systems on board the AUV. In order to accomplish data processing, the AUV retrieves and analyzes data through the use of sensors and processors. The sensors detect sound, light/color, shape, and position. The processor retrieves the inputs from the sensors and outputs the correct commands to the components of the AUV to successfully compete in the AUVSI Foundation & ONR's 14th International RoboSub Competition. In addition to data processing the AUV must be able to maneuver underwater. This was accomplished by placing a battery and inertial measurement unit (IMU) along with other water sensitive components in a watertight hull, and by using waterproof thrusters to move. Finally, the AUV must perform mission tasks such as dropping a marker and locating and hoisting a rescue object.

## Batteries

The batteries on board the AUV power all the systems for a minimum of 15 minutes in order to compete. They power the processor, all sensors, the thrusters, marker dropper, and mechanical grabber.

We chose a High Power Polymer Lithium-Ion battery to power the thrusters because they have a maximum voltage of 14.8V, a maximum capacity of 20Ah, and a maximum current draw of 30A. The batteries are connected in series to provide 29.6V, which is within the allowable range of the voltage regulator on the thrusters. This battery was selected in order to give the AUV a run time of 1 hour at maximum amp so that we have an extended period of time to work on debugging and system calibration during the testing phase. A Lithium-Iron Phosphate battery was also considered for the AUV; however, it was more expensive and was considerably heavier. Nickel Metal Hydride batteries could not supply sufficient Amp hours while Nickel Cadmium batteries do not have sufficient voltage or amp hours. In order to provide power for the processor and sensors, a voltage divider is used to drop the 29.6V from the batteries to 5V.  For risk associated with the batteries see section 2.5.1.6.2.

## Microcontroller

The microcontroller analyzes all inputs from the sensors on the AUV and output the correct commands to the thrusters, marker dropper, and mechanical grabber in order to function competitively. The microprocessor receives input from the cameras, hydrophones, and inertial measurement unit. An ARM processor has been donated to the project and was used in order to cut cost associated with the design. For risks associated with the microcontroller see sections 2.5.1.6.1 and 2.5.1.6.6.

## Adruino Board

The Adruino PWM Board serves to create a pulse width modulated signal for required apparatus. The board is capable of many different speeds and has 13 channels for output.

## Hydrophones

Our device utilizes four SenserTec SQ2601 hydrophones which will locate an acoustic pinger that will be somewhere in the Camp Transdec anechoic pool. This will be accomplished by using these hydrophones in the configuration shown in Figure 1. This configuration allows for the maximum amount of space between all of the hydrophones, thus the maximum interpretable delay between sound waves. To determine the location of the pinger the bottom two hydrophones are used to determine which direction the hydrophones are facing, left or right. After they have determined the direction, the thrusters reorient the vehicle such that it is facing the horizontal direction of the pinger. Once the vehicle is facing the appropriate direction, the device utilizes the left and/or right set of vertical hydrophones to identify when the vehicle has reached the vertical level of the pinger. At this point the vehicle follows the sound emitted by the pinger, continuously checking to ensure that it is still heading in the correct direction. Once the pinger location is reached the cameras are utilized to identify the required task.

Figure 1: Hydrophone orientation

The SensorTec SQ26-01 hydrophone was chosen for our AUV because it performs in the required range of the pinger (22-40 kHz) and is relatively cheap, when compared to other hydrophones. These hydrophones are priced at $200 each for a total of $800 for all hydrophones, which is within our budgeted amount. These hydrophones were based on three major factors: durability, cost, and frequency range. Although these hydrophones did not have as large of a frequency range as alternative hydrophones, they did operate in our required range. However the alternative hydrophones tripled and quintupled the cost of the SensorTec hydrophones, making them undesirable. Table 1 show our decision matrix comparing two other hydrophones that were considered, the Reson TC4013 and CR1 hydrophones.

| Criteria | Reson TC4013 | CR1 | SensorTec SQ26-01 | Weight |
| --- | --- | --- | --- | --- |
| Cost | 1 | 3 | 5 | .50 |
| Durability | 5 | 4 | 4 | .25 |
| Frequency Range | 5 | 4 | 3 | .25 |
| Weighted score (out of 5) | 3 | 3.5 | 4.25 |  |

Table 1: Decision Matrix

## Cameras

Three cameras are placed on the AUV in order to detect light/color and shape of objects so that the AUV will know in which direction to travel and which mission task it is presented with.

The original cameras that were chosen were the Unibrain Fire-I webcams for the AUV because it has a CCD lens, which allows it to operate in low light conditions. They ranged from $100-$130 and were going to be used for shape and color recognition in the upward, forward and downward configuration. Unfortunately, they were not UVC compatible. Due to this fact, the Unibrain Fire-I webcams could not be used.  With further research, the computer engineers decided on the LogiTech QuickCam Pro 4000 webcam because it is Linux compatible, which is the operating system used for the AUV, and because it is an inexpensive CCD lens camera. Most other CCD lens cameras cost $500-$1000 and the budget could not withstand this expense. For risk associated with the cameras see sections 2.5.1.2.1 and 2.5.1.6.5.

### Camera Housing

The camera housing is used to protect the cameras from water damage. The housing is made from 4 inch O.D. acrylic pipe. The viewing end cap is made of a ½ inch acrylic disk to allow clear images. The end cap on the back is made from a ½ inch aluminum disk. The end caps and pipe are threaded to allow the components to screw on and off. Nitride O-rings are used to provide a watertight seal. We require three camera housings and each one cost $30 for materials. This was within our estimation of $30-$40 each. The machine shop assisted with the manufacturing of the housings. For risk associated with the camera housing see section 2.5.1.1.2.

## Inertial Measurement Unit

An inertial measurement unit is used to detect changes in the AUV orientation while underwater. IMUs typically include a combination of gyroscopes and accelerometers. The IMU outputs a signal to the processor, which determines whether the thrusters need to accelerate/decelerate, pitch up/down, or turn the AUV left/right.

A PhidgetSpatial 9 Axis inertial measurement unit was chosen for the AUV because it has a 3-axis gyroscope, accelerometer, and compass. An alternative route was to purchase an accelerometer and compass unit and a separate gyroscope, which would cost $130. The more expensive PhidgetSpatial 9 Axis IMU was chosen because it contained all the desired components in a single unit, which was easier to work. The PhidgetSpatial 9 Axis IMU cost $150, which is the amount we set aside for purchasing an IMU. For risks associated with the IMU see sections 2.5.1.6.3 and 2.5.1.6.4.

## Hull

The hull is a Pelican Box, which is a watertight case that is large enough to house all of the water sensitive components on the AUV. Utilizing a Pelican Box is simpler than and just as effective as designing a watertight housing. By using a Pelican Box we reduce the risk of leaks developing throughout the testing phase and during transportation. A model 1450 Pelican Box with interior dimensions of 14.62”x10.18”x6.00” was chosen. This model had enough space for the batteries and other electronic components along with extra room for additional components in the future. Any excess buoyancy provided by the box is counteracted by the ballast compartments.

### Frame

The frame of the AUV is constructed out of 80/20 T-slotted aluminum framing. This allows for easy adjustability since we cannot design for all possible requirements and scenarios in the future. The 80/20 framing is structurally sound and can support all components that make up the AUV. In addition, the design of 80/20 naturally mitigates vibrations, which will reduce noise in the hydrophones. The hull and most major components will be located inside the frame to protect them and provide a compact, efficient design.

### Ballast

The ballast for the AUV is composed of sealed PVC pipes and lead weights strategically placed on the frame to keep the vehicle stable and level. The ballast is modular in case there are any additions to the unit or changes in positions of components.

### Waterproof Connectors

In order to prevent water intrusion to the electronic components, many different methods were researched. One option is to use premade waterproof connectors, such as SubConn or Fischer Connectors. These options are very expensive when you take into account how many connections would be required for every component to work successfully.

Two different methods were chosen to provide waterproof connections. The method used for the cameras consists of brass fittings being screwed into the pelican box and camera housing. The threads will be wrapped with Teflon tape to provide a seal. The brass fittings have a bard on one end, and a small diameter vinyl tube will be used to connect the two waterproof containers. This allows the usb cable to run the entire length in air between the two containers. The method chosen for the hydrophones and servos is similar, but the wire leads coming from each of those components is already waterproof. Due to this fact, a vinyl tube will come out of the pelican box and the waterproof wires will go into the tubing and pelican box. Epoxy will be injected into the tubing to provide a watertight seal between the wires and the tube. The thrusters will be waterproofed in a similar fashion as the hydrophones and servos, only with a larger diameter tube to compensate for the four larger gauge wires used to control them.

## Thruster

Four thrusters will be placed on the AUV in a configuration to provide forward/reverse, left/right turning, and depth control. Two of the thrusters will be placed vertically in the front and back, while the other two are placed horizontally on the port and starboard side. The processor and motor controller will be used to provide pulse width modulation to control the speed of each thruster.

The SeaBotix SBT150 Thruster (see Appendix D for data sheet) was chosen because of its functional ability and water resistance, the fact that it has a built in motor controller and voltage regulator, and for its low power consumption—view Table 2 to compare these factors with the other thruster options. The input voltage for the voltage regulator is 22-30V to continuously supply the motor with the required 19V. The SeaBotix BTD150 was also considered for this project but was not chosen because it lacked a motor controller/voltage regulator and was otherwise equivalent to the SBT150. For risk associated with the thrusters see section 2.5.1.2.3.

Table 2: Thruster Options

|  | Cost | Thrust | Power Consumption | Dry Weight | Rank |
| --- | --- | --- | --- | --- | --- |
| Weighting Factor | 0.2 | 0.2 | 0.5 | 0.1 |  |
| Seabotix BTD-150 | 7 | 6 | 9 | 9 | 8 |
| Crust Crawler 400HFS | 5 | 10 | 4 | 10 | 6 |
| Technadyne 260 | 6 | 8 | 5 | 8 | 6.1 |

## Marker Dropper

In order to perform competitively at the next AUVSI competition, a marker dropper was designed for the AUV. The marker dropper is machined out of aluminum and utilizes a water proof servo motor that rotates clockwise or counterclockwise to release individual markers. The Traxxas 2056 High-Torque waterproof servo motor was the most cost effective water proof servo. See Figure 2 for a CAD drawing of the marker dropper. The first design had the marker dropper positioned in the middle of the AUV underneath the hull. The marker dropper was moved to the inside and front of the vehicle and placed directly next to the camera facing down. This provided protection from the frame and provided a higher probability of the marker dropping successfully in the bin since it is closer to the camera.

Figure 2: Marker dropper

## Mechanical Grabber

The mechanical grabber is used to hoist a rescue object to the surface. An array of claws attached to servo motors will be used to grasp the object. The conceptual design is shown in Figure 3. Due to time constraints, the team had to focus on the primary functions and components of the AUV. In previous years, many teams were not able to successfully grab the object and bring it to the surface. The team decided not to complete this component before the end of the semester in order to complete the other components.

Figure 3: Array of Claws

# Test Plan

To ensure the effectiveness of our vehicle and its components our group performed tests on independent units as well as integration between. As a part of unit testing we evaluated the hydrophones, power systems, camera, inertial measurement unit, power systems, marker dropper, and pelican box. The thermocouples and motors have not yet undergone testing due scheduling setbacks but efforts to test components are planned. Two tests were also deemed unnecessary due to extenuating circumstances. Some components failed after undergoing tested, reasoning for failure was determined and efforts to retest these components are underway.

## System and Integration Test Plan

Since not all of the components have been tested or have failed in some component of testing, integration testing has not been completed for all integrations. The major integration factors to be tested will be pelican box connections, microprocessors and sensors, power system and thrusters, and the RoboSub.

The major system integration tests are for the microprocessor communication and RoboSub integration.  Since all of the devices will be required to communicate with the microprocessor we have begun testing the integration of these devices with the microprocessor to ensure effective communication. These tests have been initiated between sensors and the BeagleBoard but have not been completed, wee Appendix B for full test procedure and results.

Once all of the devices and parts have undergone unit testing, the entire device will be constructed, integrating all of the components. At this time full testing will begin to determine the overall effectiveness of the device. Full scale debugging will also be accomplished during this stage of testing. See Appendix B for full test procedure.

## Test Plan for Major Components

### Batteries

The power source was and passed all tests to ensure that the current capacity as well as the lifespan was suitable for our vehicle operating conditions. Power consumption was simulated using the motors well above our anticipated conditions to ensure that the batteries are able to provide sufficient power for a worst case scenario.  During this testing the effectiveness of the integration between the batteries and motors was established. See Appendix B for full test procedure and results.

### Cameras

A single camera was tested to ensure that it worked effectively with the OpenCV software, which will be utilized for image processing. The test was modified due to linking errors in the OpenCV software. After some modification the camera obtained a pass on this test. The other two cameras were not tested due to financial constraints; once the remaining two cameras are acquired they will also be tested to ensure that they are also effective at producing quality images. See Appendix B for full test procedure and results.

### Hydrophones

The hydrophones underwent one test ensure that they were manufactured properly. They were tested inside of a container filled with water. Since a sound source loud enough to propagate through the container was not available they were tested by taping on the side of the container, which produced pressure waves necessary to output voltage. This modification obtained the hydrophones a pass on this test. At this time only two of the four hydrophones were tested, the other two will be tested when they are obtained. One of the tests was deemed unnecessary, specifically a test to determine the hydrophone positioning. The positioning was limited due to other devices occupying space in front of the vehicle; the hydrophones were positioned such that there was the least amount of obstruction due to other devices. Full scale testing in the FSU pool will be completed when an underwater sound source with the necessary sound level is obtained. See Appendix B for full test procedures and results.

### Humidity Module

This component was deemed unnecessary for testing the pelican box, thus it was not purchased or calibrated. See Appendix B for initial test procedure and other comments.

### Inertial Measurement Unit

The inertial measurement unit (IMU) underwent two phases of testing: one on a Windows Operating system and another on the Linux operating system. The IMU passed both phases of testing and successfully identified movements of the device. See Appendix B for full test procedure and results.

### Thermocouple

The thermocouple testing has not been completed because the electronics have not yet been integrated. The thermocouple will also undergo testing so that it can be accurately calibrated. This initial calibration will be performed by introducing the device to an environment where the temperature is known so that the output voltage can be correlated to the accurate temperature. The thermocouple will be used to determine the temperature inside of the pelican box after 15 minutes of operation. See Appendix B for full test procedure.

### Marker Dropper

This test is incomplete; the marker dropper was successful during testing in air but has yet to be tested water due to time constraints. The marker dropper will be tested in water to ensure that there are no leaks present that will affect the performance. Ultimately the dropper will also be tested in the pool environment to ensure optimal performance. See Appendix B for full test procedure.

### Motors

The motors were not able to be tested due to programming delays as well as insufficient I2C interfaces on the boards, an I2C expansion board was purchased for the BeagleBoard to accommodate all of the motors and efforts are being made to complete programming for testing purposes on available interfaces on Arduino Duemilanove board. The motors will be tested to ensure that they were manufactured properly and can perform at desired capacity when underwater. Finally they will be tested to ensure that all thrusters work well when integrated together. See Appendix B for full test procedure.

### Pelican Box

The pelican box passed waterproof testing after having holes drilled through casing and waterproofing measures had been taken. The box was then placed in the bottom of a pool and later examined to ensure that no water had entered the casing. See Appendix B for full test procedure.

### Waterproof Connections

The waterproof connections failed during testing to ensure that they maintained a water resistant barrier after waterproofing measures were taken.  After examination it appeared that the epoxy did not seal correctly thus allowed a path for water to enter the vinyl tubing. A different method of waterproofing will be approached and the test will be conducted again.  See Appendix B for full test procedure, results, and comments.

### Voltage Regulator

The voltage regulator successfully output 5 amps at a steady rate of 5.28 V. It sustained this magnitude for 45 minutes with no cooling assistance aside from natural convective cooling from ambient air. See Appendix B for full test procedure and results.

## Summary of Test Plan Status

As of now, almost all of the components have been tested with the exception of certain units which require additional components to be obtained and/or require additional testing. The results and reasoning for failure or incompletion for all of the tests are documented in the test plan documents, which can be found in the Appendix B. Table 3 displays a summary of our testing results.

Table 3: Testing results and comments

| Test | Result | Comments |
| --- | --- | --- |
| Batteries | Pass |  |
| Voltage regulator | Pass |  |
| Pelican Box | Pass |  |
| Motors | Incomplete | Programming issues, also awaiting arrival of I2C expansion board |
| Marker Dropper | Incomplete | Passed testing in out of water conditions, pool testing will commence as soon as possible |
| Thermal Couple | Incomplete | Awaiting integration of all internal electronics |
| Humidity Module | N/A | Will not be utilizing humidity module |
| Inertial Measurement Unit | Pass |  |
| Cameras | Pass | One of three cameras was tested and passed, awaiting arrival of other two cameras to commence with testing |
| Hydrophones | Incomplete | Hydrophones passed first two phases of testing, pool testing will commence when necessary sound source is obtained |
| Arduino Duemilanove: Integration testing | Incomplete |  |
| BeagleBoard: Integration Testing | Incomplete |  |

# Schedule

CONCEPTUAL DESIGN REVIEW SCHEDULE

UPDATED SCHEDULE

One of the biggest risks that Team 4 feared was scheduling risks. At the beginning of the project, each member was aware of the complexity of the project and journey that was ahead of them. After the first semester was completed, the team realized that they were behind schedule and needed to pick up the pace. Unfortunately, some situations, such as new information, linking errors and compatibility issues, were not expected. As the software engineers were diligently working on the programming of the motors—they received new information about the motors that they were not aware of when the motors were purchased. The motors did not come with a user manual and requesting one took time. Several members had to sign a non-disclosure agreement in order to have a user manual sent to them. When the user manual arrived, it was then realized that I2C communication was needed. The software engineers were not familiar with this type of communication and they had to put the programming of the motors on hold. The software engineers immediately did research on this type of communication and unfortunately found out that they had to order an additional board to accommodate it. The board has not arrived and has caused a major delay in having the motors running. The other setbacks included linking errors in the OpenCV software and compatibility issues with cameras. Besides actually moving the AUV, the next biggest component of the programming was the cameras. The software engineers were not able to resolve linking errors within the software until recently. Code was written, but was not able to be executed until the software was up and running. Now that the software is running, the code was compiled, but did not run as it was suppose to. Besides linking errors, the software engineers had to deal with compatibility issues. The cameras purchased were suppose to be compatible with Windows Vista, but ended up having issues with the operating system. Due to this, instead of using a laptop—a desktop computer was used, resulting in the reinstallation of the OpenCV software.

Besides those unexpected delays, the team experienced delays in equipment delivery, machinery delays, waterproof connection issues, and the team’s academic course loads as well as working schedules. Some of the team’s equipment delivery dates were not met, which caused several setbacks.  Several of the components on the RoboSub were taken to the machine shop located at the college. Unfortunately, it took a long time for the machine shop to complete those components. The mechanical engineers were still able to build the RoboSub to the best of their abilities so it could be shown off at the Design Fair. The last setback was the team’s academic course load and working schedule. Each team member was considered as full students having at least 12 credit hours during the full term of this project. On top of that, some members had jobs on the side as well. Unfortunately, this factored into the time each member had to work outside of school on the project. Since the competition is not until July, the members will be able to focus solely on this project after the semester is over to ensure they provide an exceptional product to their customers.

# Final Budget and Justification

| AUV ESTIMATED BUDGET(Proposal) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Expenses (S&H Estimated) |  |  |  |  |
| Name | Description | Price | Quantity | Total |
| Battery A | Batteries for motors | $800.00 | 2 | $1,600.00 |
| Battery B | Batteries for microcontroller and other peripherals | $100.00 | 4 | $400.00 |
| Battery Charger A | Charger for battery type A | $180.00 | 1 | $180.00 |
| Battery Charger B | Charger for battery type B | $60.00 | 2 | $120.00 |
| Motors/Thrusters | Seabotix BTD150 thrusters | $750.00 | 4 | $3,000.00 |
| Hydrophones | Hydrophone package(includes hydrophone, amps, and cables) | $320.00 | 3 | $960.00 |
| Microcontroller** | Beagle Board | $170.00 | 1 | $170.00 |
| Camera | Webcam | $80.00 | 3 | $240.00 |
| Pelican Case | Water tight casing | $150.00 | 1 | $150.00 |
| Wires/Electronic Kits | Switches, wires, electronic kits | $750.00 | N/A | $750.00 |
| 30’’ 8020 10 Series Aluminum T-slot Framing | Frame | $ 14.00 | 4 | $56.00 |
| 7.75’’ 8020 10 Series Aluminum T-slot Framing | Frame | $4.00 | 4 | $16.00 |
| 8020 10 Series Aluminum T-slot Framing | Frame | $ 5.50 | 4 | $22.00 |
| 8020 Corner Bracket | Connects T-slot frame | $5.00 | 16 | $80.00 |
| Aluminum Plate 14 in x 12 in x ¼ in | Frame | $70.00 | 1 | $70.00 |
| 8020 Corner Braket Fasteners (Screw & simple nut) | To assemble frame | $1.00 | 64 | $64.00 |
| 8020 Component Fasteners (Screw & Curved Nut) | To attach components to frame | $1.50 | 40 | $60.00 |
| Inertial Measurement Unit | Navigation/Stability Control | $150.00 | 1 | $150.00 |
| Total Expenses | Total estimated cost of all expenses |  |  | $8,088.00 |
| Total Funds | Total estimated amount of sponsorships |  |  | $9,500.00 |
| Total Remaining Balance | Total estimated remaining funds |  |  | $1,412.00 |

| AUV CURRENT BUDGET |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| EXPENSES |  |  |  |  |  |
| NAME | DESCRIPTION | PRICE | QUANTITY | PURCHASED? | TOTAL |
| Harris Corporation | Donation | N/A | N/A | N/A | +$2500.00 |
| Arm, ltd. | Donation | N/A | N/A | N/A | +$2000.00 |
| Northup Grumman | Donation | N/A | N/A | N/A | +$3000.00 |
| Harris Corporation | Donation | N/A | N/A | N/A | +$2000.00 |
| Motors/Thruster | Seabotix SBT 150 | $721.07 | 4 |  | -$2884.29 |
| Frame | Hpe Automation | $220.68 | N/A |  | -$220.68 |
| Main batteries: batteryspace.com | Connector/adaptors, cord fuse 10amp & 20amp,awg wires red, green & black 14 & 18 awg, li-ion battery 14.8v, shipping chgs | $920.25 | N/A |  | -$920.25 |
| IMU | Trossen robotics:iphidgetspatial-inertial measurement unit | $162.23 | 1 |  | -$162.23 |
| Hydrophone | Cetacean Research Technology (2) hydrophone with 30cm cable | $203.33 | 2 |  | -$406.66 |
| Miscellaneous ECE items | pc cables ($76.23) misc cables:6 ft, 30 in, usb to db9-m serial port generic, usb to ethernet 10/100, usb 2.07 port powered hub&walmart ($42.75); team #4 amazon.com 3m super vinyl electrical tape, gorilla glue; fuses, power distribution, diode, on/off switch, fuse holder, I2C Expansion Board, Hercules Switching Regulator, usb mini A Female to Female A | $118.98 +$12.71 +$25.02 +$12.32 +$57.97 +$23.65 | N/A |  | -$250.65 |
| Miscellaneous ME items | Mcmaster-carr:clear acrylic circle, tube, multipurpose aluminum alloy, dash #238, o-ring two sizes, dash #341,stainless steel ball,socket,screw,thread 8-32, spacers; mcmaster-carr: acrylic circle 1/2" thick 4:dia, acrylic tube, aluminum 4" dia, o-ring, stainless steel ball, steel button head socket cap, spacers; offshore electronics inc: (1) servo arms super strength; High torque servo motor, Metal Plates (internal frame) | $127.75 +$127.03 +$16.84 +$22.25 +$76.25 +$168.61 | N/A |  | -$538.73 |
| Pelican Box | Ralph's industrial electronic supplies: waterproof box | $87.40 | 1 |  | -$87.40 |
| Web Camera | (Logitech Quickcam pro4000 web camera) | $144.98 | 3 |  | -$413.96 |
| Microcontroller | BeagleBoard**, Arduino Board | $0.00 +$31.44 | 1 |  | -$31.44 |
| Total Expenses |  |  |  |  | -$5916.29 |
| Total Funds |  |  |  |  | $9500.00 |
| Total Remaining Balance |  |  |  |  | $3583.71 |

## Budget Justification

This is the current budget including all of the costs and donations that transpired during the development of the FAMU/FSU College of Engineering AUV. The budget format is a little different but there are few changes from our proposed budget, some items were completely removed from our design and some of items were a little bit cheaper than estimated. As we proposed earlier our main control system will be centered around the BeagleBoard, a single board computer that has an OMAP3530 (TI) processor which is based on the ARM Cortex A-8. The BeagleBoard that we will use has been donated to us and meets the ARM requirement, which states that all computing equipment must be an ARM product. We have added a smaller much cheaper microcontroller, the Arduino Duemilanove; the Arduino will be used to communicate with the Hydrophones by using the ADCs. The motors have also remained the same we are using SeaBotix SBT150s that include both motor controllers and thrusters with shroud surrounding the propellers. The motor controllers communicate via I2C so an I2C expansion board was purchased because the Beagleboard does not have any I2C pins and the Arduino board only has one set of I2C pins. Originally we had proposed to power the BeagleBoard using two different sets of batteries but we found it more convenient to using voltage /current regulators with the motor batteries to power everything else. This method allowed us to save a few dollars. All of the electronic component will be housed in a pelican box as stated in the original budget but the actual price is a lot lower then estimated. The web camera (color and shape detection), hydrophone (sound detection) and IMU (roll, pitch and yaw) are also items that have not changed but their prices have changed because the estimated amount did not match the actual amount. All items were chosen based on whether they were compatible with Linux, since our computer is a Linux embedded system. The frame as well as the miscellaneous parts such as spare aluminum and electronic kits is listed on the current budget; only this time they are all listed under titles in the budget specific to their function. The budget has remained the same for the most part with the exception of a few things being added and some others being removed. On a final note we must also keep in mind that things may change between now and July (the month of the competition).

# Conclusion

Conclusively, Team 4 has made great progress and is continuously working and researching to gain the upper hand on the competition. As mentioned, Team 4 has redefined intended goals, and finalized the design of the AUV along with potential alternatives, just in case some goals need to be altered for unexpected reasons. Team 4 has already purchased and/or received the bulk of the major components, including framing, motors/thrusters, hydrophones, and cameras, just to name a few. Progress in the software aspect of the project is excelling. In that aspect, the operating system is Linux and the programming languages used are C and C++. The software engineers are working with the available sensors as well as prioritizing so the best product can be provided to the customer.

Since this project’s first purpose is for the educational assessment of a team of engineering students, a hardware demonstration is scheduled to show the progress made over the last six months. Realistically, the AUV will be operational to the best of the team’s knowledge, but due to schedule setbacks and other issues, the project is behind its desired status. For the hardware demonstration, the AUV will not be at its maximum functional status, but the team is still working hard to have the AUV ready for the competition.

Regardless, the project, as a whole, is proceeding well and the team is dedicated to satisfying the customer at all cost. Team 4 is capable of producing an exceptional product to compete in the AUVSI competition. The expected result of this project is to create a vehicle, which not only meets the design requirements, but also does so in a way, which is more precise and efficient than our competitors. Team 4 is in the process of achieving this outcome. The system meets all requirements and constraints that have been set by the preliminary rules. Hard work and devotion makes Team 4 a worthy candidate for providing the complete satisfaction of the requirements associated with the competition as well as additional objectives as time permits.

# References

Official Rules for 2010 competition:

"Official Rules and Mission AUVSI & ONR's 13th Annual International Autonomous Underwater Vehicle Competition." AUVSI Foundation. Web. Sept.-Oct. 2010. <http://www.auvsifoundation.org/AUVSI/FOUNDATION/UploadedImages/AUV_Mission_Final_2010.pdf>.

Preliminary Rules for 2011 competition:

<https://campus.fsu.edu/courses/1/EEL4914C.sp11.web_cohort1/content/_4213146_1/PreliminaryMission_2011_RoboSub.pdf?bsession=156246546&bsession_str=session_id=156246546,user_id_pk1=526216,user_id_sos_id_pk2=1,one_time_token=>

Barngrover, Chris. "Design of the 2010 Stingray Autonomous Underwater Vehicle." AUVSI Foundation. Office of Naval Research, 13 July 2010. Web. 09 Nov. 2010. <http://www.auvsifoundation.org/AUVSI/FOUNDATION/UploadedImages/SanDiegoiBotics.2010JournalPaper.pdf

# Appendix A – User’s Guide

- System Description

|  | Component | Characteristics |
| --- | --- | --- |
| 1 | Frame | Aluminum Framing & Brackets |
| 2 | Body | Pelican Box |
| 3 | Motors | SBT-150 Thrusters |
| 4 | Marker Dropper | Aluminum, Servo Motors, Markers |
| 5 | Camera Housing | Acrylic cylinder and viewing lens, O-ring, and aluminum end cap |
| 6 | Internal mounting frame | Aluminum |
|  | Mounting Brackets | All components mounted to frame have customized aluminum mounting brackets |
|  | Tubing | Vinyl tubing |

# Major Components

# Frame

The frame will support all the systems for the AUV. It is assembled with 80/20 Aluminum and anchor fasteners. Every component will be attached to the AUV via nuts and bolts to allow for easy adjustability. The frame dimensions are 24in x 22in x 7.5in

- Pelican Box

The purpose of the pelican box is to keep all water sensitive components dry when the AUV is submerged. It is depicted in the Pro-E model without a top so that some of the internal components can be seen. The Pelican box external dimensions are 16.5in X 15in X 7in, the internal dimensions are 14.875in x 10.5in x 6in.

a.    Internal Frame

The internal frame is mounted inside the pelican box and supports all the water sensitive electronics (microprocessors, IMU, voltage regulators, etc.), as well as the batteries that power the AUV. It is fabricated out of aluminum and is raised 0.5 inches off the bottom of the Pelican Box so that in the event of a minor leak the AUV has time to surface before the electronics are submerged in water. The batteries are strapped in with Velcro which run through the slots that have been cut into the internal frame, and the electronics are mounted to the base.

- Camera Housing

The camera housing is fabricated out of an acrylic cylinder, an acrylic viewing lens, and an aluminum end cap. There in an O-ring between the end caps and the acrylic cylinder, this keeps water from leaking in. The aluminum end cap has a hole so that wires can pass from the webcam to the microprocessor for communication. The end cap is sealed with resin so that it is impermeable to water.

  - Camera

The Logitech QuickCam Pro 4000 communicates directly with the BeagleBoard and allows the microprocessor to identify physical obstacles and interpret which tasks are required to be performed.

- Marker Dropper

The marker dropper is powered by a water proof servo motor that rotates a swing arm clock-wise and counter-clockwise. The semicircular guide rails keep the red markers from shifting within the aluminum housing.

- Thrusters

The Seabotix thrusters are another top of the line part within the AUV robot. The thrusters allow the AUV to move under water. They are powered by the batteries inside the Pelican Box and are controlled via a voltage regulator which sends a pulse width modulated signal to vary the speed of the motors. The motors will receive an input anywhere from 20V-30.1V DC and consume a maximum of 6A. The motors run off of I2C technology. The actual thruster motor runs off of 19.1V DC, but due to the built in internal motor controller, the required input voltage is the higher aforementioned voltage. See the above diagram for the pin out.

- Hydrophones

The hydrophones are required to locate a pinger at the bottom of the pool at Camp Transdec, CA. There are four hydrophones which communicate with the microprocessor to identify the direction and distance to the pinger.

- Lithium Ion Battery (14.4V)

The two Lithium Ion batteries are top of the line batteries with a very high current charge density. The batteries come with a built in Protection Circuit Module (PCM), an important component of the batteries. The PCM prevents overcharge from the provided charger, as well as current discharge from the battery. The batteries have a limit of 40 Amps continuous discharge, any more than that and you will shut off the batteries which a complete charge needed to reset. The PCM also will automatically shut off the battery when the power dissipates to only 10V left. Lastly, the batteries need 30 minutes after a charge before being used. The PCM uses this time to even distribute the charge across the cells and it is pertinent that this is followed.

- Beagleboard Microprocessor

The BeagleBoard is a low-cost, fan-less single-board computer based on TI's OMAP3 device family, with all of the expandability of today's desktop machines, but without the bulk, expense, or noise. It uses a TI OMAP3530 processor (ARM Cortex-A8 superscalar core ~600MHz paired with a TMS320C64x+ DSP ~430MHz and an Imagination SGX 2D/3D graphics processor).  The BeagleBoard has DVI-D, JTAG, SD/MMC+, Stereo In, Stereo Out, USB OTG, RS-232 Serial, 128MB LPDDR RAM, 256MB NAND flash. The board can only be programmed by first booting the board with an operating system. This process is down by downloading the files for an operating system and putting them on an SD card. The user must perform a dual-partition, booting from a FAT 32 file system and utilizing a Linux EXT3 file system as the root file system. For this project we chose to use the Angstrom distribution. The instructions to perform these steps are online. The board can now be programmed by using a cross-compiler in which you will compile programs on your host computer and executable/bin files will be produced to run on the BeagleBoard. The executable file must be transferred to the root file system of the BeagleBoard and then the programs can be run by producing make files to run them. Most of the information for using the BeagleBoard can be found online.

- Arduino PWM Board

The Arduino Duemilanove is an inexpensive, small microcontroller based on the ATmega328. It has 14 digital I/0 pins (6 can be used for PWM), 6 analog inputs, a 16MHz oscillator, a USB connection, a power jack, an ICSP header, and a reset button. The board operates at 5V but recommends an input of 7-12 volts because it has a built in voltage regulator. The board is programmed using the Arduino IDE which can be downloaded from the Arduino website, http://www.arduino.cc/en/Main/ArduinoBoardDuemilanove. The Arduino Duemilanove comes preburned with a bootloader that allows you to upload new code to it without the use of an external hardware programmer. It communicates using the original STK500protocol. You can also bypass the bootloader and program the microcontroller through the ICSP (In-Circuit Serial Programming) header. The Arduino software comes with numerous C libraries that make programming it very simple for a user of any skill.

- Hercules Switching Voltage Regulator

The Hercules Switching Voltage Regulator (SVR) is also commonly known as a Battery Eliminating Circuit (BEC). This SVR will take an input of anywhere from 15-60V DC and switch is down to one of 4 voltages. It can output 4.1V, 5.3, 6.1, or 8.3 volts. There are two flip switches on the SVR which yields four combinations and chooses one of the available outputs. The BEC will output up to 5 amps for more than 2 continuous hours and requires virtually no heat dissipation due to the fact that it is a switching voltage regulator, as opposed to a traditional regulator which burns off excess voltage through heat. This SVR steps the voltage down by turning on and off many times in one second which yields over a 75% efficiency. For determining the output voltage with the flip switch, 00 yields the lowest voltage, 01 the second lowest, 10 the third, and 11 yields the highest output voltage.

- Accessories

- Lithium Ion Battery Charger

There are two supplied battery chargers, one for each battery. Each charger requires 14 hours to charge a battery. This is the reason there are two chargers instead of one.

- BeagleBoard Power Supply

This power supply supplies conventional USB 5V 1.0A DC power to board.

- USB-to-Ethernet Adapter

The USB-to-Ethernet converts USB port to Ethernet for enhanced internet capabilities.

- 8GB SD Card

An SD card is used for expansion and driver loading with BeagleBoard.

- Diodes

There are 4 diodes in the system to prevent back EMF’s which often occur when the motors are turned off. The diodes are rated for a maximum of 6 amps and 100 Volts DC.

- SPDT Switch

The switch is a typical Single Pole Double Throw switch. The switch is rated for a maximum of 5 amps.

- Barrel Fuse Holder

There are 4 barrel fuse holders. These will hold fuses safely up to 60 amps. That said, the wire going into the fuse holder is only 16 gauge, so plan accordingly.

- 8 - Terminal Wire Connection Block

The connection block serves as a distribution block for the system. There are 8 pairs of individually separated terminals. Use this as needed; this team used it for a distribution block to power the motors.

- USB Divider

Powered HUB divides 1 USB port into 4 for ease of access and multi-use.

- Bill of Materials

- Nuts and Bolts

  - Roll in T-nuts

  - ¼ 20 Screw

  - Anchor Fasteners

  - Marker dropper screws

  - Pelican box screws

  - Internal frame nuts and bolts

- Mounting Brackets

  - Motor side forward mount (x2)

  - Motor side rear mount (x2)

  - Motor end mount (x2)

  - Hydrophone mount (x4)

  - Forward camera housing mount (x1)

  - Vertical camera housing mount (x2)

  - Pelican Box mount (x2)

  - Internal frame angle bracket (x4)

  - Internal frame base (x1)

- 80/20

  - 6.5 in (x4)

  - 18 in (x4)

  - 22 in (x4)

- Water Proof Housings

  - Pelican Box (x1)

  - Camera Housing (x3)

    - Acrylic cylinder (x3)

    - Aluminum end cap (x3)

    - Viewing lens (x3)

- Electronics

  - Lithium Ion Battery 14.4V (x2)

  - Seabotix SBT-150 Motor (x4)

  - Sensor-Tec SQ26-01Hydrophones (x4)

  - Logitech QuickCam Pro 4000 (x3)

  - Arduino PWM Board (x1)

  - BeagleBoard Microprocessor (x1)

  - Phidget Spatial 3/3/3 Inertial Measurement Unit (x1)

  - Hercules Switching Voltage Regulator (x1)

- Electronics Accessories

  - Lithium Ion Battery Charger (x2)

  - BeagleBoard Power Supply (x1)

  - USB-to-Ethernet Adapter (x1)

  - 8GB SD card (x1)

  - Diode (x4)

  - Barrel fuse holder (x4)

  - SPDT Switch (x1)

  - 8 - Terminal Wire Connection Block (x1)

  - 14-Gauge Wire (~10 ft)

  - 18-Gauge Wire (~20 ft)

- Specifications

  - Thrusters (4)

    - The thrusters are SeaBotix SBT150’s and are modified BTD150 thrusters with a voltage regulator and motor controller included.

    - The working voltage is +19.1V DC ±10% with a maximum power drain of 110W. The max amperage is 5.8A (30 second duration). Maximum continuous amperage is 4.25A.

    - Peak bollard thrust is 6.4 ft/lbs with a continual bollard thrust of 4.85 ft/lbs.

  - Waterproof Servo (1)

    - The servo is a Traxxas 2056 high-torque waterproof servo. The input voltage is 5.0-6.0V DC. The specified torque is 80.0 oz-in with a speed of 0.23 sec/60°.

  - Batteries (2)

    - The batteries used have an output of 14.6V each and will be connected in series to provide 29.2V DC. The capacity is 20 amp-hours each with a maximum current draw of 30A.

  - Size

    - The overall size of the AUV is 30” long, 26” wide, and 8” tall.

  - Weight

    - The dry weight of the AUV is approximately 60 lbs.

# Operating Instructions

Since this device is completely autonomous, the only instructions required for operation is how to turn the device on. Once the device is cut on and placed in a body of water—the user is not required to do anything else. To turn the device on, an on/off switch is attached on the right side of the RoboSub (if you are looking at the front of the RoboSub) or on the left side if you looking at the RoboSub from the back. If the switch is in the down position—it is off. All you need to do is flip the switch in the upward direction, which will turn on the whole RoboSub. After the RoboSub is successfully turned on, at least two people need to lower it into a body of water and watch it go.

- Basic Maintenance Guide

  - Pre-dive Checks

    - Camera Housings

      - Check all O-rings for damage before each submersion

      - Check housing and vinyl tubing for cracks and proper seal

      - Clean camera housing front viewing cap

    - Clean hydrophones

    - Check  hose clamps and Velcro ties on frame

    - Make sure all fasteners are tight and will not come loose

  - Post-dive Checks

    - Rinse thoroughly with fresh water regardless if submerged in a pool or salt water

    - Disconnect and remove batteries from pelican box

    - Position unit in a way to help with drying, i.e.: slanted to one side

  - Batteries

    - After running the AUV, the batteries should be charged within 1 day of use. If the batteries have not been used for more than 1 month, recharge before using.

    - To recharge, connect the batteries to the charging unit, then plug into the wall. The LEDs on the charging unit will indicate when the batteries are fully charged.

    - Do NOT leave batteries charging unattended. The batteries may become hot and have the potential to start a fire. Place batteries and charger on a fire-resistant surface when charging.

  - How to lift & transport

    - Do not try to lift the AUV by yourself. Always have two or more people to help carry. Lift with your knees and not your back, the unit is heavy.

    - The type of transportation affects the method of storage. For driving in a personal car, see “How to store assembled unit” below. If the unit is being shipped, some disassembly is required. All components that are protruding past the external frame should be removed. This includes all thrusters, the front-facing camera housing, and the hydrophones. The AUV should be placed in an extremely padded box with each removed component individually wrapped and protected.

  - How to store assembled unit

    - Disconnect  and store batteries in a secure location

    - Remove & secure markers

    - Secure external wires & tubing

    - Place in protective padded box

  - Modifying Internal Components

    - Removal of frame: to remove frame, open the pelican box and tilt the frame forwards, then lift vertically out. To replace, set down frame vertically and press down.

    - Removal of circuit boards from frame: unscrew board from standoffs.

    - Recharging of silica: place in oven on a cooking sheet (this sheet cannot be used for food after this) and heat at 300°F for 3 hours. Attached link is a helpful reference: http://www.ehow.com/way_5311385_homemade-desiccant-packs.html

  - Tools required

    - A small tool kit is included and kept with the AUV with extra fasteners.

    - All external frame screws are either #8-32 or ¼-20 stainless steel with a Phillips head slot. The screws for the thrusters are stainless steel (#6), which has a Phillips-head.

    - The hose clamps for the hydrophones and camera housings require a flat head screw driver.

    - The fasteners for the 80/20 frame require a hex wrench.

# Appendix B – Complete Test Reports

Scheduled Test Reporting Form

Test Item: Batteries

Tester Name: Kevin Miles					Tester ID No: kdm06d

Test Date: 3/25/2011					Test No: 1

Test Time:   4:30 P.M. 					Test Type: Pass/Fail

Test Location: College of Engineering Lab			Test Result: Pass

Test Objective:

The objective of this test is to test the batteries current capacity and lifespan. We will test for the current draw as well as the amp-hours against the given specs to ensure accuracy

Test Description/Requirements:

Requirements:

- SBT 150 Motors (4)

- Lithium Ion 14.4V Batteries

- 14 Gauge wire

- BeagleBoard

- Hercules Switching Voltage Regulator

- Timer

Process:

In this test, the AUV team will simulate real time power usage from the Switching Voltage Regulator and the SBT Motors. The Motors will be run at 70% (50% is the estimated needed power) and the simulation will be timed. This will give us a “worst-case” maximum amount of time we will be able to run our machine.

Anticipated Results:

It is believed that the AUV will be able to sustain power for over 1 hour. At max capacity, the system runs almost exactly 1 hour if the device is exactly as specified.

Requirement for Success:

The battery simulation must run for at least 1 hour to be considered a success.

Actual Results:

The batteries PASS the test on their power duration. The motors were run at 70% for one hour and 15 minutes. When disconnected, the battery still registered 23.4V left. Because the required voltage for the motors is greater than 20V, it is estimated that the batteries will continue to run at this speed for an additional 25 minutes. The result of this test, again, is a PASS.

Reason for Failure:

N/A

Recommended Fix:

N/A

Other Comments:

Scheduled Test Reporting Form

Test Item: Hercules Voltage Regulator (WRL-HBECHC)

Tester Name: Kevin Miles					Tester ID No: kdm06d

Test Date: 3/25/2011					Test No: 1

Test Time: 3:30 P.M.   					Test Type: Pass/Fail

Test Location: College of Engineering Lab			Test Result: Pass

Test Objective:

The objective of this test is to test the Switching Voltage Regulator (SVR). The test will analyze two important criteria of the switching voltage regulator, the current and the voltage output.

Test Description/Requirements:

Requirements:

- 28V Lithium Ion Batteries (Fully Charged)

- Voltage and current meter

- Hercules Voltage Regulator (WRL-HBECHC)

- 14 gauge wire

Process:

First, the output voltage of the SVR will be tested. The batteries will be connected to the VCC and GND inputs of the SVR the output VCC and GND will then be connected to a volt meter. The output voltage will be recorded.

Second, the output current will be tested. The batteries will be connected as before. Then, the output VCC of the SVR will be connected to a 1 ohm, 5W rated resistor, which is then connected to the positive input of the ammeter, then the negative input of the ammeter will be connected to the output ground of the SVR. Here, the ammeter will read the current passing through the circuit and ensure that 5 amps can be supplied from the SVR.

Anticipated Results:

We anticipate that the voltage regulator will output 5.3 V, and a current of 5 amps is achievable. Laboratory test from the company state that the SVR can sustain its maximum output for over an hour, the AUV group anticipates the same.

Requirement for Success:

The requirement for success is for the SVR to correctly output its stated output voltage and current levels for a sustained amount of time.

Actual Results:

The SVR was successful in its testing. It correctly output 5 amps at a steady output rate of 5.28V. The SVR sustained this amount for 45 minutes with no cooling assistance aside from the ambient air.

Reason for Failure:

N/A

Recommended Fix:

N/A

Other Comments:

Scheduled Test Reporting Form

Test Item: Autonomous Underwater Vehicle (as a whole)

Tester Name: Victoria Jefferson	/ Whole Team		Tester ID No: 5174

Test Date: TBA 						Test No: 2.5

Test Time:   TBA						Test Type: Test

Test Location: Pool near College of Engineering		Test Result: TBA

Test Objective:

The objective of this test is to verify the completed RoboSub can operate autonomously, complete a mock obstacle course within 15 minutes and can remain underwater without losing power or suffering from water damage.

Test Description/Requirements:

Requirements:

- The RoboSub

- A body of water (salt or fresh water)

- Stop Watch

- Laptop

- Someone that can swim (possibly a trained diver)

Process:

This test is the last test that will be done after the RoboSub has been completed. This test will be similar to a mock competition.  The whole team will be present and will help lower the AUV in the water. Depending on if a fresh water or salt water pool is used for the duration of testing, corrosion is another problem that we would need to keep an eye on. When the RoboSub is powered and in the water on its own, all of its components will be tested. The Motors/thrusters will be tested to make sure they can move the RoboSub through the water. The batteries will be tested to make sure they can continue to produce power to the RoboSub and the other electronics. The microprocessor will be tested to make sure the RoboSub can operate autonomously and collect the data from all of the sensors (cameras, IMU and hydrophones) and process it accordingly. All of the sensors will be tested to make sure they can detect color, objects and sound underwater. To test of all of these components, a mock obstacle course will be created such as the one that will be in the actual competition. A diver will be underwater with the RoboSub throughout the duration of this test to monitor its action and if need be, pull it back to the surface. The kill switch will also be tested by the diver.

Anticipated Results:

The RoboSub should operate on its own, maintain power throughout the test, all components should operate as previously tested, and complete the tasks without any damage (water or physical). It is not expected to operate perfectly, but if the RoboSub can pass this final test—it would be a great accomplished for everyone involved.

Requirement for Success:

The minimum requirements for success are operating autonomously, realize different tasks, and maintain power throughout the duration of it being underwater and those components within the RoboSub remain dry. It is also desired, but not required, that the RoboSub completes all tasks in the specified time frame.

Actual Results:

This test has not been completed due to schedule setbacks. This test is scheduled to be completed by the hardware demonstration.

Reason for Failure:

N/A

Recommended Fix:

N/A

Other Comments:

It is not required to accomplish all tasks in the competition, even though it is desired. Since this is the first year that FAMU/FSU RoboSub will be entered into the competition, settle steps forward are anticipated.

Scheduled Test Reporting Form

Test Item: Cameras

Tester Name: Victoria Jefferson			Tester ID No: 5174

Test Date: 3/8/2011				Test No: 2.5

Test Time: 1:00 pm   				Test Type: Test

Test Location: College of Engineering Lab		Test Result: Pass

Test Objective:

The objective of this test is to verify that all of the cameras are operational and capable with the OpenCV software.  After that objective is verified, cameras will be tested to verify that they are capable of taking still pictures and videos.

Test Description/Requirements:

Requirements:

- 3 Logitech Quick Pro

- USB power supplies

- Computer

Process:

Each camera will be tested separately to verify the test objective and later all cameras will be tested as an integration test. For each camera, the USB power supply will be connected. The OpenCV software will be opened and verified that there is a connection. Tester will take a series of still pictures as well as videos.

Anticipated Results:

Each camera will successfully connect using its USB power supply to the microprocessor, monitor with the keyboard and capture still pictures as well as videos.

Requirement for Success:

The minimum requirements for success are to be able to configure with the other components and software as well as produce reasonable quality pictures and videos.

Actual Results:

Instead of using the Fire-I webcams, we chose to use the Logitech webcams. Unfortunately, when the camera arrived and was ready for testing—the webcam was not compatible with my Windows operating system and there were linking errors in the OpenCV software. Due to this fact, I had to modify the test for the webcams. I installed the software that accompanied the camera onto a desktop computer and connected the webcam. The ultimate requirement for success was reached—the webcam is capable of producing reasonable quality pictures and videos.

Reason for Failure:

N/A

Recommended Fix:

In order to complete the original test plan, the linking errors must be resolved first. Once that is resolved, the compatibility issues will be addressed.

Other Comments:

Due to money constraints, only one webcam was purchased at the time of this test. The money for the remaining two webcams is being raised at this time. It is unknown if they will be ordered and/or arrive before this test plan is turned in. The microprocessor was not used for this test either because the code that will be used to analyze the data coming from the cameras was not complete.

Scheduled Test Reporting Form

Test Item: Inertial Measurement Unit (IMU)

Tester Name: Victoria Jefferson			Tester ID No: 5174

Test Date: 3/7/2011				Test No: 2.5

Test Time:   3:00 pm				Test Type: Test

Test Location: College of Engineering Lab		Test Result: Pass

Test Objective:

The objective of this test is to verify that the inertial measurement unit (IMU) operates according to the product manual.

Test Description/Requirements:

Requirements:

- IMU

- USB power supply

- Laptop

Process:

The IMU will be connected with all of the required pieces. Since there are no sample test programs written for Linux, the IMU will first be tested on the Windows operating system. The current version of the Phidget library will be installed and a sample test program will be run to verify that IMU is operational. After that is verified, the library will be uninstalled and the IMU will tested to see if it maintains the functionalities on the Linux operating system as it did on Windows.

Anticipated Results:

The IMU should operate on the Windows operating system; produce data such as compass data, acceleration data and gyro data. The IMU should also function on the Linux operating system and produce the same data as it did before.

Requirement for Success:

The minimum requirements for success are to operate on the Windows and Linux operating systems and produce accurate data from its compass (magnetometer), accelerometer, and gyroscope.

Actual Results:

As anticipated, the IMU accomplished the minimum requirements for success. The IMU was first tested on the Windows operating system. The software for the IMU was downloaded and the software detected the IMU immediately. I followed the user manual and moved the IMU around to see if the software would pick up its changes. I ran the sample programs and they worked as described. After testing was done on the Windows operating system, I tested the IMU on the Linux operating system. I downloaded the Linux-version of the Phidget software. The IMU functions the same as it did on Windows.

Reason for Failure:

N/A

Recommended Fix:

N/A

Other Comments:

Due to time constraints, sample test codes were not written. In that case, there is a slightly higher chance of failure within the integration test.

Scheduled Test Reporting Form

Test Item: Hydrophones

Tester Name: Yanira Torres			Tester ID No: ykt07

Test Date: March 25, 2011				Test No: 1

Test Time: 5.00 pm				Test Type: Test

Test Location: College of Engineering Lab		Test Result: Pass

Test Objective:

Preliminary investigation to ensure that the SQ26-01 hydrophones were manufactured correctly, thus can output a voltage given a sound input within the 22 kHz to 40 kHz frequency range.

Test Description/Requirements:

Requirements:

- Hydrophones

- Sound source/pinger

- Voltage source

- Oscilloscope

Process:

The microprocessor and voltage source will be set up and stored in the waterproof casing to which the hydrophones will be attached through. At this time the sound source will supply sound within the frequency range (22 kHz to 40 kHz). The spacing between the sound source and the hydrophones will begin at a distance of 10 ft, the spacing will be decreased 5 ft between each test until the hydrophones/case is reached. The voltage output will be analyzed to ensure that it is identifiable by the microprocessor. If no voltage is measured an amplifier will be added to the circuitry and the previous steps will be repeated.

Anticipated Results:

The hydrophones will function properly and effectively output an identifiable voltage at all distances.

Requirement for Success:

Hydrophones must receive input frequencies (between 22 kHz and 40 kHz) and output identifiable voltage

Actual Results:

Noise could not be produced at the required sound level to produce  a loud enough sound to propagate to the sensors through the water container, thus testing was done by tapping the container of water to analyze the voltage output. When this was done there was a noticeable change in voltage was produced, however it was very small.

Reason for Failure:

N/A

Recommended Fix:

N/A

Other Comments:

From this test we determined that we would need to amplify the signal from the hydrophones before sending the signal to the board, as it is very small. This will need to be done before proceeding to the integration test with the Arduino Duemilanove board.

Scheduled Test Reporting Form

Test Item: Hydrophones

Tester Name: Yanira Torres			Tester ID No: ykt07

Test Date: N/A					Test No: 2

Test Time: N/A  					Test Type: N/A

Test Location: College of Engineering Lab		Test Result: N/A

Test Objective:

To determine optimal spacing between SQ26-01 hydrophones to produce identifiable time delay between horizontal and vertical hydrophones.

Test Description/Requirements:

Requirements:

- Hydrophones

- Sound source/pinger

- ARM Microprocessor

- Voltage source (batteries)

- Cardboard

- Analog to Digital converter

Process:

A cardboard material will be utilized to simulate the aluminum mounting plate, used to maintain spacing between hydrophones. Several cardboard simulations will be cut out, with holes the approximate size of the hydrophones, with varying spacing between horizontal (dx) and vertical (dy) hydrophones.  Spacing on these cutouts will be a maximum of 8 in for dx and 6 in for dy while having a minimum of 3 in for both dimensions. The hydrophones will be integrated and tested in each of the hydrophone configurations. The horizontal hydrophones’ response will be examined by placing the sound source horizontally at a distance of 1 ft, 5 ft, and 10 ft away on both the right and left side for each test configuration, beginning at the largest spacing and proceeding inward. The hydrophones will be integrated and tested in each of the hydrophone configurations. The vertical hydrophones’ response will be examined by placing the sound source horizontally at a distance of 1 ft, 3 ft, and 6 ft away on both the top and bottom side for each test configuration. The bottom side will be tested by flipping the cardboard mounting frame over and preceding in the same manner the top side was performed. The time delay between the two hydrophones, both horizontal and vertical, will be examined to determine whether it is large enough to be identifiable by the microprocessor. The magnitude of the time delay at all test conditions will be recorded.

dy

dx

Anticipated Results:

Maximum distance between both horizontal and vertical hydrophones will produce largest time delay. One of the configurations will provide all individual hydrophones with negligible sound interference from any other hydrophone.

Requirement for Success:

One, or several, of the configurations will provide an identifiable time delay between both horizontal and vertical hydrophone arrays.

Actual Results:

TBA

Reason for Failure:

TBA

Recommended Fix:

TBA

Other Comments:

Due to positioning of other sensors and devices on vehicle, hydrophone positioning was determined to be the most effective on outside corners of vehicle where these devices would not interfere with sound wave propagation would not be. Due to obvious limitations in positioning this test became unnecessary.

Scheduled Test Reporting Form

Test Item: Hydrophones

Tester Name: Yanira Torres			Tester ID No: ykt07

Test Date: TBD					Test No: 3

Test Time: TBD   					Test Type: Test

Test Location: FSU Outdoor Pool			Test Result: Incomplete

Test Objective:

Test the chosen configuration from the SQ26-01 Hydrophone Test No. 2 in underwater conditions to ensure its effectiveness.

Test Description/Requirements:

Requirements:

- Hydrophones

- Sound source/pinger

- ARM Microprocessor

- Voltage source (batteries)

- Waterproof Casing

- Aluminum Mounting Bracket

- Body of Water (chlorinated pool)

- Analog to Digital converter

Process:

Chosen configuration will be integrated into the aluminum mounting bracket as well hooked up to the ARM Processor and voltage source in a waterproof casing. The sound source will be placed at a depth of 16 ft in the pool. The effectiveness will be determined by placing the hydrophone configuration at various positions in the pool, using a maximum distance determined in Test No. 1. The time delay will be re-examined in underwater conditions by using a written program on microprocessor to determine whether the location of the sound source is positively identified. The hydrophones will be held vertically, horizontally (facing opposite directions), and diagonally from the sound source in a particular order. After testing is complete the directions that the microprocessor identified and recorded will be analyzed and compared to the actual directions.

Anticipated Results:

The microprocessor will successfully identify the directions given the chosen configuration.

Requirement for Success:

The microprocessor must able to positively identify all directions (horizontal and vertical) given the chosen configuration.

Actual Results:

Incomplete

Reason for Failure:

Test is incomplete due to schedule setbacks.

Recommended Fix:

Once necessary sound source is obtained and amplification circuit is established we will be able to proceed with testing.

Other Comments:

Scheduled Test Reporting Form

Test Item: BeagleBoard

Tester Name: Andy Jeanthenor				Tester ID No: aj09

Test Date: 2/01/2011					Test No: 1

Test Time: 1 PM						Test Type: Test

Test Location: College of Engineering Lab			Test Result: Incomplete

Test Objective:

The objective of this test is to test the BeagleBoard and ensure that it performs the functions needed for AUV operation properly.

Test Description/Requirements:

Requirements:

- Microprocessor

- Power Supply

Process:

The proposed battery figuration will be run in some software like MultiSim to see if the simulation produces the desired results. Once the correct circuit is built the BeagleBoard will be connected to our proposed battery configuration and the tester will check to see if it is completely operational. The BeagleBoard will be left on for a specified amount of time and the tester will check to see if the BeagleBoard works properly for the entire duration.  This process will be repeated until a passing grade is received. The ports will be tested by using the Board Verification instructions in the user manual. If the hardware is not working properly then a new board will have to be purchased since the board was donated to the project and a replacement board is most likely out of the question. The compatibility will be tested by running test software that uses the sensors through the BeagleBoard. Data will be recorded to determine whether proper communication was established.

Anticipated Results:

The microcontroller and the sensors are fully powered by the proposed battery configuration. The ports that will be needed for communication between the microcontroller and the sensors are operational. The BeagleBoard properly communicates with purchased sensors produces the desired results.

Requirement for Success:

The requirement for success is that the microcontroller is completely operational with the proposed battery configuration. The sensors and the microcontroller communicate properly and perform the desired functions.

Actual Results:

Incomplete

Reason for Failure:

The proposed power configuration has not been tested at the moment and the sensors have not been tested due to unforeseen circumstances. Getting the board up and running is taking a lot longer than expected. The board hardware was tested and runs exactly like it should, according the board verification instructions in the manual say it should.

Recommended Fix:

The proposed power configuration will be tested as soon as possible and integration with the sensors will take a little longer than expected; but they will both be completed as soon as possible. We are trying our best to get the board up and running but the instructions in the manual are not well written for beginners. There are instructions on the internet but they are usually in bits and pieces. So it is time consuming getting the board to work.

Scheduled Test Reporting Form

Test Item: Arduino Board

Tester Name: Andy Jeanthenor				Tester ID No: aj09

Test Date: 2/01/2011					Test No: 1

Test Time: 1 PM						Test Type: Test

Test Location: College of Engineering Lab			Test Result: Incomplete

Test Objective:

The objective of this test is to test the Arduino Duemilanove and ensure that it performs the functions needed for AUV operation properly.

Test Description/Requirements:

Requirements:

- Microcontroller Operation

- Sensor Integration

- Communication with the BeagleBoard

Process:

The Arduino Duemilanove will be tested for factory hardware verification by reading the manual and make sure everything works as it should in the manual. The ports will then be tested by using sample code with LEDs as output to make sure the pins work properly. If the pins work properly the Arduino Duemilanove will be used to analyze the signals that are picked up by the Hydrophones by using the ADC pins on the Arduino. The data will then be sent to the BeagleBoard where the data will be analyzed and used to complete certain tasks. The Arduino board will also be used to control the Servo motors that will be used to operate the marker dropper and this configuration will be tested using sample code as well.

Anticipated Results:

The board will light the required LEDs indicating that both a power and data connection has been established. The test code will run and show that the pins work properly. The hydrophone will receive data, the ADCs will convert the data and the data will be sent to the BeagleBoard.

Requirement for Success:

The requirements for success include that the Arduino board works according to factory verification instructions. Also, the PWMs should light the LEDs and run the servo motors using the sample code. The last requirement for success is that the Arduino board reads the data from the hydrophone and analyzes the data.

Actual Results:

Incomplete

Reason for Failure:

The Arduino Board has run properly and the sample code has been tested so we know the pins work. We are only waiting to test the hydrophones.

Recommended Fix:

Once the hydrophones are tested if they pass the test then this test will be complete otherwise we will have to consider other options.

Other Comments:

Scheduled Test Reporting Form

Test Item: BeagleBoard with I2C expansion

Tester Name: Andy Jeanthenor				Tester ID No: aj09

Test Date: 2/01/2011					Test No: 3

Test Time: 1 PM						Test Type: Test

Test Location: College of Engineering Lab			Test Result: Incomplete

Test Objective:

The objective of this test is to test the BeagleBoard and ensure that it performs the functions needed for AUV operation properly.

Test Description/Requirements:

Requirements:

- I2C expansion

- BeagleBoard

- Motors

- Power Supply

Process:

The I2C expansion board will be connected to the BeagleBoard as well as the laptop, which will act as the power supply. The BeagleBoard needs to realize the presence of the I2C board and sample code will be ran to see if the appropriate input is relayed to the BeagleBoard as well as the correct output is sent to the motors.

Anticipated Results:

The expansion board connects properly with BeagleBoard and the BeagleBoard recognizes the expansion board during the boot sequence. Upon running the code the BeagleBoard sends the appropriate instructions to the motors.

Requirement for Success:

The requirement for success is that the I2C is recognized by the BeagleBoard and properly performs the desired tasks.

Actual Results:

Incomplete

Reason for Failure:

The I2C expansion board has not yet arrived

Recommended Fix:

Once the I2C expansion board arrives it will be connected and tested, if it does not work there will be harsh consequences.

Other Comments:

Scheduled Test Reporting Form

Test Item: Microcontroller with Sensors

Tester Name: Andy Jeanthenor				Tester ID No: aj09

Test Date: TBD						Test No: 4

Test Time: TBD						Test Type: Test

Test Location: College of Engineering Lab			Test Result: Incomplete

Test Objective:

The objective of this test is to test whether the microcontroller and the sensors perform the functions needed for AUV operation properly.

Test Description/Requirements:

Requirements:

- Microprocessor

- Sensors

- Cameras

- IMU

- Hydrophones

- Power Supply

Process:

The microcontroller will have to be integrated with all the sensors to make sure they are all working together properly. The sensors will be programmed individually and tested individually to make sure each sensor works independently of the other sensors. The integration will be tested by running software that will use all the sensors at the same time and will simulate AUV operation. The position, depth, pressure, temperature and humidity sensors will have to be tested during the test runs. This will be tested by storing the data received from the sensors and using a laptop to remotely monitor sensor activity. The cameras will be tested using a laptop as a host to observe if the software is accurately retrieving the correct data from the cameras. The motor controllers will be tested by using a motor control module of the software and observing the motors. The test runs will be documented and all errors will be addressed. This process will be repeated until all sensors are working properly with the microcontroller. Simulation tools will be used prior to physical testing to save time.

Anticipated Results:

The microcontroller properly communicates with purchased sensors and produces the desired results.

Requirement for Success:

The requirement for success is that the microcontroller is completely operational with the sensors and the microcontroller communicates properly and performs the desired functions.

Actual Results:

Incomplete

Reason for Failure:

The BeagleBoard is not up and running yet nor had the power system been verified

Recommended Fix:

Once the BeagleBoard is up and running; the sensors will be connected and tested.

Other Comments:

We are severely behind schedule

Scheduled Test Reporting Form

Test Item: Pelican Box

Tester Name: Tadamitsu Byrne			Tester ID No: trb06e

Test Date: 04/05/2011				Test No: 1

Test Time:    7 PM				Test Type: Test

Test Location: Reece's Pool			Test Result: Pass

Test Objective:

The objective of this test is to determine whether the pelican box is water tight to a depth of 15 feet after modifications have been made to the original box. The inside of the box was inspected for leaks.

Test Description/Requirements:

Requirements:

1- Pelican Box

2- Pool

4- Brass fitting

Process:

A hole is machined in the pelican box that allows wires to pass in and out. A brass fitting was placed in the hole to seal the box for this test. The pelican box was then filled with weights to submerge it and allowed to sit at the bottom of the pool for 15 minutes.

Anticipated Results:

The inside of the pelican box is expected to be dry after 15 minutes of being submerged.

Requirement for Success:

The pelican box is water tight and no leaks are evident.

Actual Results:

The pelican box did not leak.

Reason for Failure:

N/A

Recommended Fix:

N/A

Other Comments:

N/A

Scheduled Test Reporting Form

Test Item: HM1500LF Humidity Module

Tester Name: Tadamitsu Byrne			Tester ID No: trb06e

Test Date: N/A					Test No: 1

Test Time:  N/A					Test Type: Test

Test Location: N/A				Test Result: N/A

Test Objective:

The objective of this test is to calibrate the HM1500LF - Relative Humidity Module.

Test Description/Requirements:

Requirements:

- HM1500LF Humidity module

- Multimeter

Process:

The HM1500LF will be introduced to an environment where the relative humidity is known. The output voltage will be measured and the %RH (percent relative humidity) will be calculated as follows:

Anticipated Results:

The HM1500LF will relay output voltage corresponding to the known RH.

Requirement for Success:

The HM1500LF indicates the correct voltage corresponding to the RH.

Actual Results:

This test was not done. It was determined that a humidity module is not required to test the pelican box.

Reason for Failure:

N/A

Recommended Fix:

N/A

Other Comments:

N/A

Scheduled Test Reporting Form

Test Item: Waterproof connections between pelican box and external hardware

Tester Name: Tadamitsu Byrne			Tester ID No: trb06e

Test Date: 04/05/2011				Test No: 1

Test Time:   7 PM					Test Type: Test

Test Location: Reece’s Pool			Test Result: Fail

Test Objective:

The objective of this test is to determine whether our custom waterproof connectors actually work.

Test Description/Requirements:

Requirements:

- Vinyl tube

- Mock wiring

- Brass fitting

- Hose clamp

- Epoxy

Process:

The custom waterproof connections are made by sealing one end of the vinyl tube with a brass fitting and hose clamp. The other end is filled with resin and has mock wiring to determine whether the tube is impermeable to water.

Anticipated Results:

The epoxy will successfully keep the water from entering the vinyl tube.

Requirement for Success:

The inside of the vinyl tube must be completely dry. Any moisture within the tube is a requirement for failure.

Actual Results:

The custom waterproof connectors allowed water inside the vinyl tube.

Reason for Failure:

The epoxy did not seal correctly and created a path for which the water could enter.

Recommended Fix:

Be more careful when applying the epoxy. Make sure the ends are completely sealed. Try a new manufacturing process.

Other Comments:

N/A

Scheduled Test Reporting Form

Test Item: Thermocouple

Tester Name: Tadamitsu Byrne			Tester ID No: trb06e

Test Date: TBD					Test No: 1

Test Time:   TBD					Test Type: Test

Test Location: College of Engineering Lab		Test Result: TBA

Test Objective:

The objective of this test is to calibrate a thermocouple to determine the temperature inside the pelican box.

Test Description/Requirements:

Requirements:

- Thermocouple

- Multimeter

Process:

The thermocouple will be introduced to an environment where the temperature is known. The temperature will be calculated based on output voltage and will be calibrated beforehand.

Anticipated Results:

The thermocouple will relay output voltage corresponding to the known temperature.

Requirement for Success:

The thermocouple indicates the correct voltage corresponding to the temperature.

Actual Results:

This test was not done because this piece of equipment was not purchased as of yet.

Reason for Failure:

TBA

Recommended Fix:

TBA

Other Comments:

This test also has not been completed because all hardware and electronics have not been integrated inside the pelican box yet.

Scheduled Test Reporting Form

Test Item: AUV Marker Dropper

Tester Name: Reece Spencer & Andy Jeanthenor	Tester ID No: rus05 & aj09

Test Date: 3/1/11					Test No: 1

Test Time:    5 PM				Test Type: Test

Test Location: College of Engineering Lab		Test Result: Incomplete

Test Objective:

The objective of this test is to test the marker dropper and ensure that it will run properly when connected to the microprocessor. The assembly must function properly in air and water.

Test Description/Requirements:

Requirements:

- Marker Dropper Assembly

- Traxxas Waterproof Servo Motor

- Arduino Microcontroller

- Voltage source (batteries)

Process:

The servo motor will be attached to the marker dropper assembly, and then connected to the processor and power supply. A program will be made to test the servo motor. The servo should rotate the shaft approximately 30° in each direction for proper functionality. The test will rotate the shaft to at least 60° each direction. After the servo is found to be working, two markers will be placed inside the assembly. The servo will be tested with the markers in place to make sure it can release each one individually. Once the whole assembly is working properly in air, the assembly will be placed in a bath of water and tested again to ensure that the dropper will work properly in the environment it will be ran in.

Anticipated Results:

The marker dropper will release each marker individually and will not leak.

Requirement for Success:

The requirement for success is that the marker dropper can successfully operate in dry and wet conditions.

Actual Results:

Incomplete

Reason for Failure:

The marker dropper is tested and works for air conditions but have yet to be tested for underwater conditions.

Recommended Fix:

Once the microcontroller and marker dropper are assembled for underwater testing this test should be complete.

Other Comments:

Scheduled Test Reporting Form

Test Item: Thrusters

Tester Name: Reece Spencer			Tester ID No: rus06

Test Date: 2/01/2011				Test No: 1

Test Time:   5 PM					Test Type: Test

Test Location: College of Engineering Lab		Test Result: Incomplete

Test Objective:

The objective of this test is to test the thrusters and ensure that they will run properly with the motor controllers when connected to the microprocessor.

Test Description/Requirements:

Requirements:

- SBT-150 Thrusters (4)

- Microprocessor

- Voltage source (batteries)

Process:

Each motor will be individually connected to the microprocessor and batteries. A program will be made to give the motors an input of 28V. An internal voltage regulator in the motors will reduce the voltage to 19V, which is their recommended running voltage. If the motors work successfully then a program will be made to use the motor controller and attempt different duty settings from 0-100% in increments of 10%. Once dry testing is completed, the motors will be checked to ensure that they are watertight, then will be placed into a small water bath and tested again to make sure they will work when submerged. After all motors have been tested individually, all four will be connected to the microprocessor and batteries to ensure that they will all work in unison.

Anticipated Results:

The motors will run with the anticipated duty from the motor controllers and will not leak.

Requirement for Success:

The requirement for success is that motors can successfully run in dry and wet conditions.

Actual Results:

TBA

Reason for Failure:

TBA

Recommended Fix:

TBA

Other Comments:

Due to setbacks with programming I2C the motors were not able to be tested. The team is working on finalizing programming so that the motors may be tested.

# Appendix C – Software

Describe the software developed for your system and then provide listings for the programs you have written.

Program Structure (Based on 2011 Preliminary Rules)

--Check Status--

IF status OK THEN	--Check to see if microcontroller properly communicating with sensors

Begin Mission

ELSE			--if complete communication is not established abort the mission

Abort Mission (deploy emergency shutdown mechanisms)

END IF

Begin Mission:

--Validation Gate--

DO { 			--continue to search for the validation gate until it is found

Read data from the camera/iMU

Locate validation gate

Compute data needed to pass validation gate

} WHILE   Gate is not found

Send Data to Motor Controllers		--after gate is found clear the gate

Drive through validation gate

Go to the Path

Path:		--this code designates what is to be done when the AUV encounters the path

DO {		--the AUV must first find the path

Read data from the Camera/IMU

Locate the path (Orange PVC Sheet)

Compute data needed to follow the path

} WHILE   Path is not found

Send data to motor controllers	--now that the path is found follow it

Follow the path

Read Value of N 	--Now that the path has been found go to the task

Go to task that corresponds to the current value of N

--N will be used as a counter to find out which task to execute

Flowers: --N = 0 this code designates what is to be done when the AUV encounters the flowers

DO {		--find the buoys which represent the flowers

Read data from the Camera/IMU

Locate the buoys

Compute data needed to get in striking range of buoys

} WHILE  Buoys not found

Send data to motor controller --now that the buoys are found AUV can proceed to strike them

Strike the buoys

N = N + 1			--increment the task counter

Go to the path

LoveLane: --N = 1 this code designates what is to be done when the AUV needs to cross the “L”

DO {	--find the L shaped PVC pipe

Read data from the Camera/IMU

Locate the “L”

Compute data needed to cross the top of risers

} WHILE  “L” not found

Send data to motor controllers	--now that the risers are found AUV can cross them

Cross the top of the “L”

N = N + 1			--increment the task counter

Go to the path

LoveLetters:	--N = 2 this code is for when the AUV needs to drop markers

DO {		--find the bins which house the letters

Read data from the Camera/IMU

Locate the correct bin

Compute data needed to hover over bin 1

} WHILE  Black bin 1 not found

Send data to motor controllers	--now that the bin is found hover over it

Drive AUV over black bin 1

DO {		--find the correct bin

Read data from the Camera/IMU

Locate bin 1

Compute data needed to drop marker

} WHILE  Correct bin not found

Send data to motor controllers	--now that the bin is found the AUV can drop the markers

Drop both markers		--both markers will be dropped into one bin to simplify the task

N = N + 1			--increment the task counter

Go to the path

Cupid:	--N = 3 this code designates what is to be done when the AUV needs to rescue the object

DO {		--find the acoustic pinger

Read data from the hydrophones

Analyze the signal and locate the pinger

Compute data needed to get to pinger

} WHILE  Pinger not found

Send data to motor controllers	--now that the pinger is found drive to it

Drive AUV over pinger

DO {		--find the rescue object which represents the counselor

Read data from the Camera/IMU

Locate the box of candy

Compute data needed to reach and gab box of candy

} WHILE  Box of Candy not found

Send data to motor controllers	--now that the box of candy is found the AUV can grab it

Grab and hold box of candy

DO {		--rise to the top of the pool with the box of candy

Read data from the Camera/IMU

Locate the octagon and its boundaries

Compute data needed to rise within the octagon

} WHILE  Octagon not found

Send data to motor controllers	--now that the octagon is found the AUV rise to the top

Rise to the top of the pool within the octagon shape

Release the box of candy

STANDBY	--end of pseudo code

# Computer Software Structure

# Computer Hardware Structure

# List of Programs Written

- Main program

  - The purpose of the main program will be used to call for input from each sensor. Depending on the input received from the sensor, the program will need to be able to send out the correct output.

- Sensor programs

  - Cameras

    - The purpose of the camera program will be to detect colors and shapes. For color recognition, the wavelength of the colors will be analyzed to determine which color it is. If the correct color is determined then the RoboSub should be able to do a given task.

  - IMU

    - The IMU program will enable the IMU to output a signal to the processor, which determines whether the thrusters need to accelerate/decelerate, pitch up/down, or turn the AUV left/right.

  - Hydrophones

    - The purpose of the hydrophone program is to determine the location of an acoustic pinger. The program will determine if a particular hydrophone has a stronger sound intensity then others. If so, then the motors will need to move the RoboSub in the direction of the sound. The program will keep looping until the acoustic pinger is located and in sight by the cameras.

- I2C communication program

  - The I2C program is currently in the R & D phase so no further information can be given at this time. We are currently awaiting an expansion board that will allow for us to send signals from the BeagleBoard to the motor controllers.

- PWM

  - The PWMs will no longer be used for the main motors because it has come to our attention that the motor controllers communicate via I2C. The servo however will be controlled via PWMs. The Arduino Software has a built in library which controls the Servo motors, called Servo.h. This library includes a function in which you declare a servo much like you declare an integer, simply by naming it. Then there is an attach function that associates the servo motor to a PWM pin. Then there is a write function which takes in values from 0 to 180 which represent the motion of the servo arm which is up to 180 degrees. Here is an example of what the program will look like because it is not complete as of this moment.

# Appendix D – Data Sheets

# Appendix E – Qualifications/Resumes

Team four was comprised of six exceptional Computer, Electrical and Mechanical seniors from the FAMU-FSU College of Engineering. The members of team four had an opportunity to face an exciting challenge. With their impressive knowledge and skills, along with their undeniable dedication and persistence—this project could not be in better hands. Below is a table of responsibilities that each member was in charge of for the 2010-2011 academic year. Also attached are the resumes of each member for your convenience for future employment.

| Task | Assignment | Skills and Knowledge |
| --- | --- | --- |
| Project Manager, Documentation Secretary, Sensor Programming | Victoria Jefferson | Object-Oriented Programming, Data Structures, Assembly Language, Electronics, Microprocessors |
| Technical Lead, Electronics, Power | Kevin Miles | Electronics, Power Systems |
| Financial Secretary, Electronics, Microprocessor Programming/ PMW | Andy Jeanthenor | Data Structures, Assembly Language, Microprocessors, Electronics |
| Motors, Power, Propulsion System, Buoyancy Control | Reece Spencer | Mechanical Systems, Dynamic Systems, Boat Repair, Diving Experience |
| Mechanical, Propulsion System, Construction | Tad Byrne | Mechanical Systems, Thermal Fluids, Dynamic Systems |
| Mechanical, Sensors, Debugging | Yanira Torres | Sensors, Mechatronics |

Yanira Torres

1000 W Brevard St. APT 013 ۰ Tallahassee, FL 32304 ۰Phone Number (850) 363-2135 ۰ykt07@fsu.edu

Education

Florida State University 								   Tallahassee, FL

Bachelors of Science in Mechanical Engineering:  May 2011 	  		              Overall GPA: 3.69

Masters of Science in Mechanical Engineering:  May 2012

Relevant Skills

Finite Element Analysis in COMSOL, C Programming, MATLAB, Pro/ENGINEER, Autodesk Inventor, Microsoft Office Suite, SAS, MathCAD.

Experience

FAMU-FSU Mechanical Engineering Department			                 Tallahassee, FL

▫Teaching Assistant 					  			 08/2010-Present

- Responsible for educating 106 students on basic concepts in MATLAB, Pro/ENGINEER, and Working Model as they apply to analysis of mechanical systems.

- Responsible for assessing student progress as well as maintaining and organizing student records.

FAMU-FSU Active Structures and Microsystems Laboratory 		   	   Tallahassee, FL

▫Research Assistant								 08/2009-10/2010

- Conducted mathematical modeling to determine light induced strain in polymers using MATLAB.

- Characterized photoresponse of light actuated materials.

Air Force Research Laboratory		                         Wright-Patterson Air Force Base , OH

▫ Summer Intern       [Azimuth Corporation]						 05/2010-07/2010

▫ Summer Intern      [Universal Technology Corporation]				 05/2009-08/2009

- Synthesized photoresponsive liquid crystal polymers and explored structure-property relationships.

- Utilized dynamic mechanical analyzer (DMA) to determine mechanical properties and measurements.

Florida Center for Advanced Aero Propulsions				   Tallahassee, FL

▫ Research Assistant								 08/2008-05/2009

- Assisted with experiments in aeroacoustics in the High Temperature Supersonic Jet Facility.

- Gained experience with Autodesk Inventor to design structures and LabVIEW to run experiments.

- Purchased parts/materials and assembled structures for experiments.

Presentations

“Photoresponsive Azobenzene Liquid Crystal Polymer Networks: In Situ Photogenerated Stress Measurements”. American Society of Mechanical Engineers: Smart Materials, Adaptive Structures and Intelligent Systems Conference. 1 October 2010

“Photoresponsive Active Structures.” American Chemical Society: Central Regional Meeting of the American Chemical Society Conference. Dayton, Ohio.” 18 June 2010

Activities

2009-Present 	Tau Beta Pi: Engineering Honor Society, Vice President

2009-Present	Engineers Without Borders, Public Relations Chair

2007-Present 	Women in Math Science and Engineering Society, Member

Languages

Spanish- Intermediate

Italian - Intermediate

| Reece Spencer Phone: (305) 395-0986 Tallahassee, FL E-mail: rusØ5@fsu.edu |  |
| --- | --- |
| Education |  |
|  | Florida State University Expected Graduation Spring 2011 Bachelor of Science Degree in Mechanical Engineering (BSME) |
| Professional/Research experience |  |
| ○ ○ | Seacamp Big Pine Key, FL Summer, 2005 – 2010 Counselor, SCUBA, Sailing, and Windsurfing Instructor Responsibilities included: teaching campers life skills, conflict resolution, risk management, administered testing, repaired and maintained high pressure SCUBA regulators Florida Center for Advanced Aero-Propulsions Tallahassee, FL Spring 2010-Present Research Assistant Assisted graduate students with research projects, learned use of subsonic wind tunnel, hot wire data acquisition, shadowgraph/schlieren techniques Teaching Assistant Supervised labs with students, reviewed information regarding fluid mechanics and heat transfer, graded quizzes and tests, maintained course website |
| Software Skills |  |
|  | Proficient in Microsoft Word and Excel Adobe Photoshop Pro/Engineer, MathCAD, MATLAB |
| Field Related/Personal Skills |  |
|  | Leadership: Managing a university student organization, completion of instructor level courses, organizing and teaching courses to both youth and adults Computer Hardware/Software: I have built my own computers and can troubleshoot problems |
| Achivements/Memberships |  |
|  | President of Florida State University Sailing Club 2007 – 2009 Member ASME Professional & Student Organization 2010-Present Member AIAA Professional & Student Organization 2010-Present |
| Certifications |  |
|  | Red Cross Life Guarding, First Aid, CPR-PR US Sailing Instructor NAUI Master and Rescue Diver NAUI SCUBA Diving and Skin Diving Instructor Sherwood SCUBA Regulator Repair Technician |

Tadamitsu Byrne		                    Contact: 786-385-6207 / Trb06e@fsu.edu

Current Address: 1505 W. Tharpe St., Apt 3334

Permanent Address: 525 NW 145 ST Tallahassee FL, 32303     Miami FL, 33168

Objective

To obtain a position as a mechanical engineer where I can apply my analytical and problem solving skills to gain valuable work experience and be an asset to your company

Education

BS in Mechanical Engineering, Florida State University, Tallahassee FL                                    		          May 2011

Technical GPA: 3.5 / Overall GPA: 3.3

Professional Experience

High Performance Materials Institute, Florida State University, Tallahassee, FL

Intern										   		   Summer 2010

- Research focused on creating a Carbon Nanotube honeycomb structure

- Learned how to fabricate Bucky Paper and carbon fiber using vacuum filtration

- Responsible for data collection and analysis

- Familiar with TA Instruments (Q800 DMA and Q500 TGA) and SEM imaging

Vector Marketing, Miami, FL

Sales Associate										              	   Summer 2008

- Coordinated meetings with potential clients

- Promoted Cut Co. products effectively and sold over $1000 in a month

- Organized my own schedule to reach monetary sale goals

- Documented client visits and products sold

Zen Angel Service LLC, West Park, FL

Apprentice							                     		       Summer 2006/2007

- Assisted the business owner with daily tasks and procedures

- Responsible  for the basic maintenance of the company van

- Learned how to use a buzz saw, sander, staple gun, and jack-hammer

- Consistently met deadlines set by employer (Refurbished 200 restaurant chairs in  a week)

- Responsible for general cleanliness of wood shop workspace

Extracurricular Activities

American Society of Mechanical Engineers

Vice President/Member					   Fall 2010/Spring 2010

- Established focused and enthusiastic executive board

- Increased membership from last year by more than 200%

- Maintaining membership by getting members involved

  - SPDC (H2Go), Mary Brogan Outreach Program

- Responsible for coordinating guest speakers, industry tours, funding

Community Service

Service for Peace/Summer of Service, Miami Dade College

Participant (500+ Hours)                                                                     	           		        Summer 2003-2005, 2007

- Contributed to my local community by doing service projects

- Assisted with cleaning and repainting a community on Le Jeune Road

- Worked with Habitat for Humanity to build houses in Miami-Dade County

- Received the Presidential Volunteer Service Award

Computer Literacy

- Pro/Engineering

- SmartSketch LE / Working Model

- MathCAD

- MatLab

- MSC Adams/Car

- Microsoft Office Suite

Kevin D. Miles

3119 Egrets Landing, Lake Mary, FL 32746

Kdm06d@fsu.edu                         	(407) 687-3995

Education

Florida State University, Tallahassee, FL	Spring 2011

Bachelor of Science

Major: Electrical Engineering

3.32 G.P.A.

Minor: Physics, Mathematics

Experiences

Autonomous Underwater Vehicle Competition, San Diego, CA

Electronics Team Lead 	August 2010 - Present

AUV from scratch competing in underwater obstacle course

Designed all vehicle power and circuitry systems

$11,000 overall team budget

Alpha Tau Omega, Leadership Development Fraternity, Tallahassee, FL

Founding President 	January - December, 2009

True Merit Honorable Mention national award winner

Lead group in 1st place overall finish in FSU Homecoming and Greek Week

Doubled size from 28 to 64 members

Effectively designed $30,000+ budget

Created detailed scholarship, membership, and public relation programs

Circuit City Stores, Inc. Lake Mary, FL

Mobile Electronics, Installation and Sales	October 2005 - August 2007

Over $100,000 sold

Led store sales for 3 consecutive and 7 total months

Taught effective sales techniques to 10-15 new employees

Activities and Interests

Activities	IEEE, FSU Study Abroad,  Alpha Tau Omega

Services and       30+ Collegiate service hours, Lawton Chiles Leadership Corps,

Awards       Emerging Leaders Conference, FSUS Playground Project

| Key Skills |  |  |
| --- | --- | --- |
| Related Coursework Digital Signal Processing Field Program Logic Devices Signals and Systems Solid State Electronics Wireless Networks Communications | Programming Languages C++Assembly VHDL Java | Software Pspice Matlab Quartus Eclipse LTspice |

Victoria Jefferson

Email: vcj07@fsu.edu

Phone: (786) 338-1776

Permanent Address: 								Local Address:

20905 NW 32nd Court								1310 Hiawatha Farms Rd

Miami, FL 33056									Monticello, FL 32344

OBJECTIVE

To obtain a challenging position as an intern within the Computer Engineering discipline and the possibility of career advancement.

EDUCATION

Florida State University Tallahassee, Florida                                      		 December 2011

Bachelor of Science in Computer Engineering

- GPA: 3.22

SKILLS

- Languages: C++, MATLAB

- Applications: MS Visual Studio, PSPICE/Multisim, Microsoft Word, Excel, Powerpoint, Outlook

- Operating Systems: Windows, Unix

RELEVANT COURSES

- Circuits 1-2, Electronics, Signals and Systems, Digital Communication, Electromagnetic Fields I

- C++, Object-Oriented Programming, Data Structures, Operating Systems

- Microprocessors, FPLDs, Computer Architecture

- Calculus 1-3, E-Math 1-2, Discrete Math

WORK EXPERIENCE

Autonomous Underwater Vehicle Tallahassee, Florida	                         September 2010-May 2011

Project Manager/Lead Engineer

- Managing team of six engineers of diverse disciplines to build an underwater vehicle to compete in the AUVSI competition in the Summer of 2011

Florida Highway Patrol Tallahassee, Florida                                 	          August 2007-Present

Part-time Call Taker

- Perform a range of telephone operational duties in a 24-hour facility

- Use a computer-aided dispatch system (CAD)

- Answer non-emergency calls for assistance

- Enter, update and retrieve information from a variety of computer systems

Southwest Regional Library Miramar, Florida                             	           December 2005-August 2007

Part-time Student Page

- Shelved library material and performs other tasks to keep the collections in order

- Used computer hardware and software to access information

AWARDS AND ACTIVITIES

Florida Bright Futures Medallion Scholarship                                     	        	              Fall 2007-Present

Freshman Incentive Scholarship                                                                        	              Fall 2007-Present

Lambda Delta Chapter of Eta Kappa Nu Honor Society-Member			Fall 2010

National Society of Black Engineers (NSBE)-Member                                                   	Fall 2010

Society of Women Engineers (SWE)-Member                                                                	Fall 2010